UHFReader288.DLL manual

V2.1

# Contents

Contents	2

1	Introduction	5

2	SDK summary	6

3	Programming interface	7

3.1	Connect/disconnect reader	7

3.1.1	AutoOpenComPort	7

3.1.2	OpenComPort	8

3.1.3	CloseComPort	9

3.1.4	OpenNetPort	10

3.1.5	CloseNetPort	11

3.2	18000-6C commands	12

3.2.1	Inventory_G2	12

3.2.2	InventoryMix_G2	15

3.2.3	InitRFIDCallBack	18

3.2.4	ReadData_G2	19

3.2.5	WriteData_G2	21

3.2.6	BlockWrite_G2	23

3.2.7	ExtReadData_G2	25

3.2.8	ExtWriteData_G2	27

3.2.9	WriteEPC_G2	28

3.2.10	BlockErase_G2	29

3.2.11	Lock_G2	30

3.2.12	KillTag _G2	32

3.2.13	StartRead	33

3.2.14	StopRead	34

3.2.15	StopImmediately	35

3.2.16	SelectCmdWithCarrier	36

3.3	Buffer operation commands	37

3.3.1	InventoryBuffer_G2	37

3.3.2	ReadBuffer_G2	39

3.3.3	ClearBuffer_G2	40

3.3.4	GetBufferCnt_G2	41

3.3.5	SetSaveLen	42

3.3.6	GetSaveLen	43

3.4	NXP Tag commands	44

3.4.1	SetPrivacyByEPC _G2	44

3.4.2	SetPrivacyWithoutEPC_G2	45

3.4.3	ResetPrivacy_G2	46

3.4.4	CheckPrivacy_G2	47

3.4.5	EASConfigure_G2	48

3.4.6	EASAlarm_G2	49

3.5	18000-6B Commands	50

3.5.1	InventorySingle_6B	50

3.5.2	InventoryMultiple_6B	51

3.5.3	ReadData_6B	53

3.5.4	WriteData_6B	54

3.5.5	CheckLock_6B	55

3.5.6	Lock_6B	56

3.6	Reader customised commands	57

3.6.1	GetReaderInformation	57

3.6.2	SetAddress	59

3.6.3	SetInventoryScanTime	60

3.6.4	SetRfPower	61

3.6.5	SetRegion	62

3.6.6	SetBaudRate	64

3.6.7	BuzzerAndLEDControl	65

3.6.8	SetAntennaMultiplexing	66

3.6.9	SetAntennaMultiplexing	67

3.6.10	SetBeepNotification	69

3.6.11	SetReal_timeClock	70

3.6.12	GetTime	71

3.6.13	SetRelay	72

3.6.14	GetGPIOStatus	73

3.6.15	SetGPIO	74

3.6.16	SetNotificationPulseOutput	75

3.6.17	SetTIDParameter	76

3.6.18	SetQS	77

3.6.19	GetQS	78

3.6.20	GetSeriaNo	79

3.6.21	SetCheckAnt	80

3.6.22	SetWorkMode	81

3.6.23	GetSystemParameter	82

3.6.24	ReadActiveModeData	83

3.6.25	SetEASSensitivity	84

3.6.26	SetMask	85

3.6.27	SetResponsePamametersofAuto_runningMode	86

3.6.28	SetInventoryInterval	87

3.6.29	SelectTagType	88

3.6.30	GetTagBufferInfo	89

3.6.31	ClearTagBuffer	90

3.6.32	GetModuleVersion	91

3.6.33	SetFlashRom	92

3.6.34	WriteRfPower	93

3.6.35	ReadRfPower	94

3.6.36	RetryTimes	95

3.6.37	SetReadMode	96

3.6.38	SetReadParameter	97

3.7.39	GetReadParameter	99

3.6.40	SetAntennaPower	99

3.6.41	GetAntennaPower	101

3.6.42	SetCfgParameter	102

3.6.43	GetCfgParameter	103

Appendix 1: Reader Error code table	104

Appendix 2: Tag ErrorCode table	105

# 1	Introduction

The SDK is designed for .net interface compatible languages, such as C# and VB.NET etc. It supports both 32-bit and 64-bit WINDOWS operation system.

By loading the "UHFReader288.DLL", host application can control the 18000-6C and 18000-6B formats readers. Host application can access multiple readers via different communication ports simultaneously.

# 2	SDK summary

| Files | Description |
| --- | --- |
| UHFReader288.dll | including all the API interfaces. |

| Programming language | Description |
| --- | --- |
| C# | Copy UHFReader288.dll to the exe file output directory and add a reference to the project |

# 3	Programming interface

## 3.1	Connect/disconnect reader

3.1.1	AutoOpenComPort

| Definition | int AutoOpenComPort(ref int Port, ref byte ComAddr, byte Baud); |  |  |
| --- | --- | --- | --- |
| Description | Automatically identify reader connected serial port and initialise serial connection to reader. This function only supports serial from COM1 to COM 9. |  |  |
| Parameters | Name | Type | Notes |
|  | port | ref int | Serial port number for reader connection. 1 - COM1; 2 - COM2, etc. |
|  | ComAddr | ref byte | Target reader address. In the case when reader address is unknown, the broadcasting address 0xFF can be used to initialise the serial connection. Once the function is called successfully, the actual address of the reader will be returned as the value of this parameter. |
|  | baud | byte | Serial baud rate. 0 - 9600 bps; 1 - 19200 bps; 2 - 38400 bps; 5 - 57600 bps; 6 - 115200 bps. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |
| Sample code |  |  |  |

3.1.2	OpenComPort

| Definition | int OpenComPort(int Port, ref byte ComAddr, byte Baud); |  |  |
| --- | --- | --- | --- |
| Description | Open the serial connection to reader |  |  |
| Parameters | Name | Type | Notes |
|  | port | Int | Serial port number for reader connection. 1 - COM1; 2 - COM2, etc. |
|  | ComAddr | ref byte | Target reader address. In the case when reader address is unknown, the broadcasting address 0xFF can be used to initialise the serial connection. Once the function is called successfully, the actual address of the reader will be returned as the value of this parameter. |
|  | baud | byte | Serial baud rate. 0 - 9600 bps; 1 - 19200 bps; 2 - 38400 bps; 5 - 57600 bps; 6 - 115200 bps. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |
| Sample code |  |  |  |

3.1.3	CloseComPort

| Definition | int CloseComPort(); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Close a specific port to disconnect with reader and release associated resource. |  |  |  |
| Parameters (N/A) | Name | Type | Orientation | Notes |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to error code table) . |  |  |  |
| Sample code | N/A |  |  |  |

3.1.4	OpenNetPort

| Definition | int OpenNetPort(int Port, string IPaddr, ref byte ComAddr); |  |  |
| --- | --- | --- | --- |
| Description | Open the TCP connection to reader |  |  |
| Parameters | Name | Type | Notes |
|  | port | int | TCP port number for reader connection. |
|  | IPaddr | string | IP address of the reader. |
|  | ComAddr | ref byte | Target reader address, or simply use broadcasting address 0xff for TCP connection. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |
| Sample code |  |  |  |

3.1.5	CloseNetPort

| Definition | int CloseNetPort(); |  |  |
| --- | --- | --- | --- |
| Description | Close the TCP connection with reader and release associated resource |  |  |
| Parameters | Name | Type | Notes |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |
| Sample code | N/A |  |  |

## 3.2	18000-6C commands

3.2.1	Inventory_G2

| Definition | int Inventory_G2(ref byte ComAddr, byte QValue, byte Session, byte MaskMem, byte[] MaskAdr, byte MaskLen, byte[] MaskData, byte MaskFlag, byte AdrTID, byte LenTID, byte TIDFlag, byte Target, byte InAnt, byte Scantime, byte FastFlag, byte[] pEPCList, ref byte Ant, ref int Totallen, ref int CardNum); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Inquire RFID tags with corresponding protocol within the effective field. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | QValue | byte | [in] | 1 byte. bit7: Statistic data packet flag; 0 – after inventory，DO NOT deliver statistic data packet of inventory process; 1 – after inventory，deliver statistic data packet of inventory process. bit5: FastID inventory indicator. This function is used to rapidly obtain tag EPC and TID, only supported by some tags from the Impinj Monza series; 0 – disable; 1 – enable. Bit4: phase flag 0 – disable;1 – enable. Bit3 ~ bit0: the original Q-value of EPC tag inventory. NOTE: 1. The setting of Q-value should follow the rule: 2Q ≈ total amount of tags within the effective field. The range of Q-value is 0 ~ 15, if other value is delivered in this field, reader will return a parameter error status in the response frame. 2. Inventory with statistic data packet is SLOWER than inventory without statistic data packet. |
|  | Session | byte | [in] | 1 byte, the Session-value of EPC tag inventory. 0x00 – apply S0 as Session value; 0x01 – apply S1 as Session value; 0x02 – apply S2 as Session value; 0x03 – apply S3 as Session value. 0xff – apply reader smart configuration (only valid in EPC inventory). Inventory for single tag or small amount of tag, S0 is the recommended setting. |
|  | MaskMem | BYTE | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte[] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte [] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | MaskFlag | byte | [in] | 1 byte, mask flag. 0 – disable mask; 1 – enable mask. |
|  | AdrTID | byte | [in] | 1 byte, entry address of inventory TID memory. |
|  | LenTID | byte | [in] | 1 byte, data length for TID inventory operation, the valid range of LenTID is 0 ~ 15. |
|  | TIDFlag | byte | [in] | 1 byte, inventory purpose indicator. 0 – EPC inventory; 1 – TID inventory. |
|  | Target | byte | [in] | 1 byte, the Target value of EPC tag inventory. 0x00 – apply Target A; 0x01 – apply Target B. |
|  | InAnt | byte | [in] | 1 byte, antenna selection for the current inventory. 0x80 – antenna 1; 0x81 – antenna 2; 0x82 – antenna 3; 0x83 – antenna 4; 0x84 – antenna 5; 0x85 – antenna 6; 0x86 – antenna 7; 0x87 – antenna 8; 0x88 – antenna 9; 0x89 – antenna 10; 0x8A – antenna 11; 0x8B – antenna 12. InAnt is 0x08 for single port reader. |
|  | Scantime | byte | [in] | 1 byte, the maximum operation time for inventory. The valid range of Scantime is 0 ~ 255, corresponding to (0 ~ 255)*100ms. For Scantime = 0, operation time is not limited. |
|  | FastFlag | byte | [in] | 1 byte, express inventory indicator 0 – disable express inventory, Target, InAnt and Scantime are NOT essential, can be set to default value 0; 1 – enable express inventory, Target, InAnt and Scantime are need to be defined. |
|  | pEPCList | byte [] | [out] | The inquired tag data, the data block follows the format stated below: EPC/TID length + EPC/TID No. + RSSI. Data of multiple tags is formed by several identical data blocks in sequence If length bit6=1.add 4 bytes phase information +3 bytes freqkhz after rssi |
|  | Ant | ref byte | [out] | 1 byte indicates which antenna had inquired a tag. 4 antenna ports reader: Every bit in Ant represents one corresponding antenna. For example, 0x04 is 0000 0100 in binary. This indicates Antenna 3 had inquired this specific tag. 12 antenna ports reader Ant value 0 ~ 11 corresponding to Antenna 1 to Antenna 12. For example, Ant = 0 represents Antenna 1 had inquired this specific tag. |
|  | Totallen | ref int | [out] | The total length of the received data stored in pEPCList. |
|  | CardNum | ref int | [out] | The total amount of tag inquired during the current inventory. Note: Totallen and CardNum are 0 for call back return. |
| Return value (int) | 0x01: inventory completed, tag data will be delivered; 0x02: inventory timeout, operation is aborted, inquired tag data will be delivered. 0xF8: error occurred in antenna check, the target antenna might be disconnected. For description on other error, refer to reader error code table. |  |  |  |
| Sample code | N/A |  |  |  |

3.2.2	InventoryMix_G2

| Definition | int InventoryMix_G2(ref byte ComAddr, byte QValue, byte Session, byte MaskMem, byte[] MaskAdr, byte MaskLen, byte[] MaskData, byte MaskFlag, byte ReadMem, byte[] ReadAdr, byte ReadLen, byte[] Psd, byte Target, byte InAnt, byte Scantime, byte FastFlag, byte[] pEPCList, ref byte Ant, ref int Totallen, ref int CardNum); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Inquire RFID tags with corresponding protocol within the effective field, and obtain specific tag data once EPC number is inquired. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | QValue | byte | [in] | 1 byte. bit7: Statistic data packet flag. 0 – after inventory，DO NOT deliver statistic data packet of inventory process; 1 – after inventory，deliver statistic data packet of inventory process. Bit4: phase flag 0 – disable;1 – enable. Bit3 ~ bit0: the original Q-value of EPC tag inventory. NOTE: 1. The setting of Q-value should follow the rule: 2Q ≈ total amount of tags within the effective field. The range of Q-value is 0 ~ 15, if other value is delivered in this field, reader will return a parameter error status in the response frame. 2. Inventory with statistic data packet is SLOWER than inventory without statistic data packet. |
|  | Session | byte | [in] | 1 byte, the Session-value of inventory. 0x00 – apply S0 as Session value; 0x01 – apply S1 as Session value; 0x02 – apply S2 as Session value; 0x03 – apply S3 as Session value. Inventory for single tag or small amount of tag, S0 is the recommended setting. |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte[] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte[] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | MaskFlag | byte | [in] | 1 byte, mask flag. 0 – disable mask; 1 – enable mask. |
|  | ReadMem | byte | [in] | 1 byte, the type of target memory. 0x00 – reserved memory; 0x01 – EPC memory; 0x02 – TID memory; 0x03 – user memory. All other values are reserved, reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | ReadAdr | byte[] | [in] | 2 bytes, the initial word address of target, most- significant byte first. |
|  | ReadLen | byte | [in] | 1 byte, the amount of word to be read in this operation. |
|  | Psd | byte[] | [in] | 4 bytes, the access password. |
|  | Target | byte | [in] | 1 byte, the Target value of tag inventory. 0x00 – apply Target A; 0x01 – apply Target B. |
|  | InAnt | byte | [in] | 1 byte, antenna selection for the current inventory. 0x80 – antenna 1; 0x81 – antenna 2; 0x82 – antenna 3; 0x83 – antenna 4; 0x84 – antenna 5; 0x85 – antenna 6; 0x86 – antenna 7; 0x87 – antenna 8; 0x88 – antenna 9; 0x89 – antenna 10; 0x8A – antenna 11; 0x8B – antenna 12. InAnt is 0x08 for single port reader. |
|  | Scantime | byte | [in] | 1 byte, the maximum operation time for inventory. The valid range of Scantime is 0 ~ 255, corresponding to (0 ~ 255)*100ms. For Scantime = 0, operation time is not limited. |
|  | FastFlag | byte | [in] | 1 byte, express inventory indicator 0 – disable express inventory, Target, InAnt and Scantime are NOT essential, can be set to default value 0. 1 – enable express inventory, Target, InAnt and Scantime are need to be defined. |
|  | pEPCList | byte[] | [out] | The inquired tag data, the data block follows the format stated below: Rolling code + EPC length/data length + EPC No. /data + RSSI. Data of multiple tags is formed by several identical data blocks in sequence. The most-significant bit of rolling code: 0 – this block contains EPC; 1 – this block contains tag memory data. Firstly EPC will be uploaded, followed by tag memory data. If two sets of lower 7 bits of rolling code are consecutive, the corresponding data comes from the same tag. The lower 7 bits of rolling code ranges from 0 to 127. The code will increase for every block uploading and will recount again from 0 after it exceeds 127. If length bit6=1.add 4 bytes phase information +3 bytes freqkhz after rssi |
|  | Ant | ref byte | [out] | 1 byte indicates which antenna had inquired a tag. 4 antenna ports reader: Every bit in Ant represents one corresponding antenna. For example, 0x04 is 0000 0100 in binary. This indicates Antenna 3 had inquired this specific tag. 12 antenna ports reader Ant value 0 ~ 11 corresponding to Antenna 1 to Antenna 12. For example, Ant = 0 represents Antenna 1 had inquired this specific tag. |
|  | Totallen | ref int | [out] | The total length of the received data stored in pEPCList. |
|  | CardNum | ref int | [out] | The total amount of tag inquired during the current inventory. Note: Totallen and CardNum are 0 for call back return. |
| Return value (int) | 0x01: inventory completed, tag data will be delivered; 0x02: inventory timeout, operation is aborted, inquired tag data will be delivered. 0xF8: error occurred in antenna check, the target antenna might be disconnected. For description on other error, refer to reader error code table. |  |  |  |
| Sample code | N/A |  |  |  |

3.2.3	InitRFIDCallBack

| Definition | public void InitRFIDCallBack(RFIDCallBack t); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Initialise call back function, after the initialisation of this API, tag inquired from Inventory and Mix Inventory commands can be uploaded via call back. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | pUIDback | RFIDCallBack | [in] | Call back interface pointer. public class RFIDTag { public byte PacketParam; public byte LEN; public string UID; public byte RSSI; public int Freqkhz; public byte ANT; public string DeviceName; }; |
| Return value | N/A |  |  |  |
| Sample code |  |  |  |  |

3.2.4	ReadData_G2

| Definition | int ExtReadData_G2(ref byte ComAddr, byte[] EPC, byte ENum, byte Mem, byte[] WordPtr, byte Num, byte[] Password, byte MaskMem, byte[] MaskAdr, byte MaskLen, byte[] MaskData, byte[] Data, ref int errorcode); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Read a part of data or the whole memory area from tag reserved memory, EPC memory, TID memory, user memory. Read operation starts from a specific address. The unit of data is word. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | byte[] | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Mem | byte | [in] | 1 byte, the type of target memory. 0x00 – reserved memory 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. All other values are reserved. Reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | WordPtr | byte | [in] | 1 byte, the initial word address of target. |
|  | Num | byte | [in] | 1 byte, the amount of words to be read in this operation. |
|  | Password | byte[] | [in] | 4 bytes, the access password. |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte[] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte[] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | Data | byte[] | [out] | Inquired tag data, with a length stated in Num. |
|  | ErrorCode | ref byte | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.5	WriteData_G2

| Definition | int WriteData_G2(ref byte ComAddr, byte[] EPC, byte WNum, byte ENum, byte Mem, byte WordPtr, byte[] Wdt, byte[] Password, byte MaskMem, byte[] MaskAdr, byte MaskLen, byte[] MaskData, ref int errorcode); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Write multiple words to reserved memory, EPC memory, TID memory or user memory in one communication cycle. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | byte[] | [in] | Tag EPC number, with a length stated in Enum. |
|  | Wnum | byte | [in] | 1 byte, the amount of words to be written. For ENum = 255, mask will be enabled. |
|  | Enum | byte | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Mem | byte | [in] | 1 byte, the type of target memory. 0x00 – reserved memory 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. All other values are reserved. Reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | WordPtr | byte | [in] | 1 byte, the initial word address of target. |
|  | Writedata | byte[] | [in] | Data to be written to tag, with a length stated in WNum. |
|  | Password | byte[] | [in] | 4 bytes, the access password. |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte[] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte[] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | ref int | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.6	BlockWrite_G2

| Definition | int BlockWrite_G2(ref byte ComAddr, byte[] EPC, byte WNum, byte ENum, byte Mem, byte WordPtr, byte[] Wdt, byte[] Password, byte MaskMem, byte[] MaskAdr, byte MaskLen, byte[] MaskData, ref int errorcode); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Write multiple words to reserved memory, EPC memory, TID memory or user memory in one communication cycle. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | byte[] | [in] | Tag EPC number, with a length stated in Enum. |
|  | Wnum | byte | [in] | 1 byte, the amount of words to be written. For ENum = 255, mask will be enabled. |
|  | Enum | byte | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Mem | byte | [in] | 1 byte, the type of target memory. 0x00 – reserved memory 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. All other values are reserved. Reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | WordPtr | byte | [in] | 1 byte, the initial word address of target. |
|  | Writedata | byte[] | [in] | Data to be written to tag, with a length stated in WNum. |
|  | Password | byte[] | [in] | 4 bytes, the access password. |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte[] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte[] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | ref int | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.7	ExtReadData_G2

| Definition | int ExtReadData_G2(ref byte ComAddr, byte[] EPC, byte ENum, byte Mem, byte[] WordPtr, byte Num, byte[] Password, byte MaskMem, byte[] MaskAdr, byte MaskLen, byte[] MaskData, byte[] Data, ref int errorcode) ; |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Read data from reserved memory, EPC memory, TID memory, user memory of tag. Read operation starts from a defined address, and the unit of data is word. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | byte[] | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Mem | byte | [in] | 1 byte, the type of target memory. 0x00 – reserved memory 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. All other values are reserved. Reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | WordPtr | byte[] | [in] | 2 bytes, the initial word address of target, most- significant byte first. |
|  | Num | byte | [in] | 1 byte, the amount of word to be read in this operation. |
|  | Password | byte[] | [in] | 4 bytes, the access password. |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte[] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte[] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | Data | byte[] | [out] | Inquired tag data, with a length stated in Num. |
|  | ErrorCode | ref int | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.8	ExtWriteData_G2

| Definition | int ExtWriteData_G2(ref byte ComAddr, byte[] EPC, byte WNum, byte ENum, byte Mem, byte[] WordPtr, byte[] Wdt, byte[] Password, byte MaskMem, byte[] MaskAdr, byte MaskLen, byte[] MaskData, ref int errorcode); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Write multiple words to reserved memory, EPC memory, TID memory or user memory in one communication cycle. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | byte[] | [in] | Tag EPC number, with a length stated in Enum. |
|  | Wnum | byte | [in] | 1 byte, the amount of words to be written. |
|  | Enum | byte | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Mem | byte | [in] | 1 byte, the type of target memory. 0x00 – reserved memory 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. All other values are reserved. Reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | WordPtr | byte[] | [in] | 2 byte, the initial word address of target. |
|  | Writedata | byte[] | [in] | Data to be written to tag, with a length stated in WNum. |
|  | Password | byte[] | [in] | 4 bytes, the access password. |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte[] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte[] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | ref int | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.9	WriteEPC_G2

| Definition | int WriteEPC_G2(BYTE * ComAddr,BYTE *Password, BYTE *EPC, BYTE ENum, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Write EPC number to a tag. During the operation, only one tag is allowed to be place in the antenna effective area. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | Password | byte[] | [in] | 4 bytes, the access password. |
|  | EPC | byte[] | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number. |
|  | ErrorCode | ref int | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.10	BlockErase_G2

| Definition | int BlockErase_G2(ref byte ComAddr, byte[] EPC, byte ENum, byte Mem, byte WordPtr, byte Num, byte[] Password, byte MaskMem, byte[] MaskAdr, byte MaskLen, byte[] MaskData, ref int errorcode); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Erase multiple words from reserved memory, EPC memory, TID memory or user memory. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | byte[] | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Mem | byte | [in] | 1 byte, the type of target memory. 0x00 – reserved memory 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. All other values are reserved. Reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | WordPtr | byte[] | [in] | 1 byte, the initial word address of target. |
|  | Num | byte | [in] | 1 byte, the amount of words to be erased in this operation. |
|  | Password | byte[] | [in] | 4 bytes, the access password. |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte[] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte[] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | ref int | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.11	Lock_G2

| Definition | int Lock_G2(ref byte ComAddr, byte[] EPC, byte ENum, byte select, byte setprotect, byte[] Password, byte MaskMem, byte[] MaskAdr, byte MaskLen, byte[] MaskData, ref int errorcode); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | The read/write protection status for the following memories: • Reserved memory Readable/ writable without protection, permanently readable/ writable, readable/writeable with password protected or permanently unreadable/ unwritable; • EPC memory /user reserved memory Writable without protection, permanently writable, writeable with password protected or permanently unwritable; EPC memory, user reserved memory and TID memory are permanently readable. Furthermore TID memory is readable only, permanently unwritable. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | byte[] | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | select | byte | [in] | 1 byte. 0x00 – modify kill password read/write protection; 0x01 – modify access password read/write protection; 0x02 – modify EPC memory read/write protection; 0x03 – modify TID memory read/write protection; 0x04 – modify user memory read/write protection. All other values are reserved, reader will not execute the command and will return a parameter error status in the response frame if other value is delivered in this field. |
|  | setprotect | byte | [in] | 1 byte. For Select = 0x00 or 0x01: the kill password or access password protection setting. SetProtect = 0x00 – set to readable/ writable without protection; 0x01 – set to permanently readable/ writable; 0x02 – set to readable/writeable with password protected; 0x03 – set to permanently unreadable/ unwritable. For Select = 0x02, 0x03, and 0x04: the EPC, TID and user memory protection setting. SetProtect = 0x00 – set to writable without protection; 0x01 – set to permanently writable; 0x02 – set to writeable with password protected; 0x03 – set to permanently unwritable. All other values of Select and SetProtect are reserved, reader will not execute the command and will return a parameter error status in the response frame if other value is delivered in this field. |
|  | Password | byte[] | [in] | 4 bytes, the access password. |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte[] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte[] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | ref int | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.12	KillTag _G2

| Definition | int KillTag_G2(BYTE * ComAddr, BYTE *EPC, BYTE ENum, BYTE *Killpwd, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Tag killing command. After the kill operation, the tags will no longer process any command from reader. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | byte[] | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Killpwd | byte[] | [in] | 4 bytes, the kill password. |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte[] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte[] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | ref int | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.13	StartRead

| Definition | int StartInventory(ref byte ComAddr, byte Target, RFIDCallBack callback); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Check whether there is an electronic label that conforms to the protocol within the valid range, this command only supports Ex10 series readers |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | Target | byte | [in] | Inventory flag，0-Target A，1-Target B |
|  | callback | RFIDCallBack | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.14	StopRead

| Definition | int StopInventory(ref byte ComAddr); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Stop fast inventory. this command only supports Ex10 series readers |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.15	StopImmediately

| Definition | int StopImmediately(ref byte ComAddr); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to stop the query command being executed. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.16	SelectCmdWithCarrier

| Definition | SelectCmdWithCarrier(ref byte ComAddr, byte Antenna, byte Session,byte SelAction, byte MaskMem,byte[] MaskAdr, byte MaskLen,byte[] MaskData,byte Truncate,byte CarrierTime); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to send a select command with carrier. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | Antenna | byte | [in] | The antenna number to be executed, 0x80~0x8F represent antenna 1~antenna 16 respectively. |
|  | Session | byte | [in] | Target parameter in the select command. 0x00~0x07 |
|  | SelAction | byte | [in] | the Action parameter in the Select command Parameter range 0~7 |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte[] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte[] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | Truncate | byte | [in] | Truncate flag. 0 - Disable. 1 - Enable |
|  | CarrierTime | byte | [in] | 1 byte, carrier on time. With 100ms as the unit, the allowable value range of the carrier time is 0*100ms~255*100ms, that is, 0~25.5 seconds |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

## 3.3	Buffer operation commands

3.3.1	InventoryBuffer_G2

| Definition | int InventoryBuffer_G2(ref byte ComAddr, byte QValue, byte Session, byte MaskMem, byte[] MaskAdr, byte MaskLen, byte[] MaskData, byte MaskFlag, byte AdrTID, byte LenTID, byte TIDFlag, byte Target, byte InAnt, byte Scantime, byte FastFlag, ref int BufferCount, ref int TagNum); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Inquire RFID tag with corresponding protocol within the effective field. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | QValue | byte | [in] | 1 byte. The original Q-value of EPC tag inventory. NOTE: 1. The setting of Q-value should follow the rule: 2Q ≈ total amount of tags within the effective field. The range of Q-value is 0 ~ 15, if other value is delivered in this field, reader will return a parameter error status in the response frame. |
|  | Session | byte | [in] | 1 byte, the Session-value of EPC tag inventory. 0x00 – apply S0 as Session value; 0x01 – apply S1 as Session value; 0x02 – apply S2 as Session value; 0x03 – apply S3 as Session value. 0xff – apply reader smart configuration (only valid in EPC inventory). Inventory for single tag or small amount of tag, S0 is the recommended setting. |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte[] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte[] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | MaskFlag | byte | [in] | 1 byte, mask flag. 0 – disable mask; 1 – enable mask. |
|  | AdrTID | byte | [in] | 1 byte, entry address of inventory TID memory. |
|  | LenTID | byte | [in] | 1 byte, data length for TID inventory operation, the valid range of LenTID is 0 ~ 15. |
|  | TIDFlag | byte | [in] | 1 byte, inventory purpose indicator. 0 – EPC inventory; 1 – TID inventory. |
|  | Target | byte | [in] | 1 byte, the Target value of EPC tag inventory. 0x00 – apply Target A; 0x01 – apply Target B. |
|  | InAnt | byte | [in] | 1 byte, antenna selection for the current inventory. 0x80 – antenna 1; 0x81 – antenna 2; 0x82 – antenna 3; 0x83 – antenna 4; 0x84 – antenna 5; 0x85 – antenna 6; 0x86 – antenna 7; 0x87 – antenna 8; 0x88 – antenna 9; 0x89 – antenna 10; 0x8A – antenna 11; 0x8B – antenna 12. InAnt is 0x08 for single port reader. |
|  | Scantime | byte | [in] | 1 byte, the maximum operation time for inventory. The valid range of Scantime is 0 ~ 255, corresponding to (0 ~ 255)*100ms. For Scantime = 0, operation time is not limited. |
|  | FastFlag | byte | [in] | 1 byte, express inventory indicator 0 – disable express inventory, Target, InAnt and Scantime are NOT essential, can be set to default value 0; 1 – enable express inventory, Target, InAnt and Scantime are need to be defined. |
|  | BufferCount | ref int | [out] | 2 bytes, the total amount of tag stored in the memory buffer, tags with identical EPC/TID data will be treated as one tag. BufferCount is the sum of tag amount from multiple inventories, until the memory buffer is being cleared. |
|  | CardNum | ref int | [out] | 2 bytes, the amount of tag inquired in the current inventory, a tag being accessed multiple times will also increase the amount. |
| Return value (int) | 0x01: inventory completed, tag data will be delivered; 0x02: inventory timeout, operation is aborted, inquired tag data will be delivered. 0xF8: error occurred in antenna check, the target antenna might be disconnected. For description on other error, refer to reader error code table. |  |  |  |
| Sample code | N/A |  |  |  |

3.3.2	ReadBuffer_G2

| Definition | int ReadBuffer_G2(ref byte ComAddr, ref int Totallen, ref int CardNum, byte[] pEPCList); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain all the tag information from reader memory. This command will not cause affection on the status of the stored data. Data can be access multiple times. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | Totallen | ref int | [out] | The total length of the received data stored in pEPCList. |
|  | CardNum | iref int | [out] | The total amount of tag inquired during the current inventory. |
|  | pEPCList | byte[] | [out] | The inquired tag data, with a length stated in Totallen. Data of multiple tags is formed by several identical data blocks in sequence, every data block follows the format stated below: ANT + LEN + EPC/TID + RSSI + Count; |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.3	ClearBuffer_G2

| Definition | int ClearBuffer_G2(ref byte ComAddr); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Clear all the stored tag data from the memory buffer. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.4	GetBufferCnt_G2

| Definition | int GetBufferCnt_G2(ref byte ComAddr, ref int Count); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain the total tag amount stored in memory buffer. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | Count | ref int | [out] | The total amount of tag stored in the memory buffer. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.5	SetSaveLen

| Definition | int SetSaveLen(ref byte ComAddr, byte SaveLen); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify the maximum EPC/TID length for inventory with memory buffer. This modification will clear all the tag data previously stored in the memory buffer. |  |  |  |
| Parameter | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | SaveLen | byte | [in] | 1 byte, the maximum EPC/TID length. 0x00 – 128bit length, i.e. 16 bytes. 0x01 – 496bit length, i.e. 62 bytes. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.6	GetSaveLen

| Definition | int GetSaveLen(ref byte ComAddr, ref byte SaveLen); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Load the configuration of maximum EPC/TID length for reader memory buffer |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | SaveLen | ref byte | [out] | 1 byte, the maximum EPC/TID length. 0x00 – 128bit length, i.e. 16 bytes. 0x01 – 496bit length, i.e. 62 bytes. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

## 3.4	NXP Tag commands

3.4.1	SetPrivacyByEPC _G2

| Definition | int SetPrivacyByEPC_G2(ref byte ComAddr, byte[] EPC, byte ENum, byte[] Password, byte MaskMem, byte[] MaskAdr,byte MaskLen, byte[] MaskData, ref int errorcode); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Setup read protection for a tag with particular EPC number; hence the tag will be unreadable by device. Device will not able to inquire EPC number of this tag via inventory operation. This command only valid for NXP UCODE EPC G2X tags. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | byte[] | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Password | byte[] | [in] | 4 bytes, the access password. |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte[] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte[] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | ref byte | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.4.2	SetPrivacyWithoutEPC_G2

| Definition | int SetPrivacyWithoutEPC_G2(ref byte ComAddr, byte[] Password, ref int errorcode); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Setup read protection for tags within the effective field. Unlike the pervious command in 3.4.1, this command will perform operation on multiple inquired tags without tag identification. In order to perform operation on multiple tags, it is vital to keep access password consistent on those tags. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | Password | byte[] | [in] | 4 bytes, the access password. |
|  | ErrorCode | ref int | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.4.3	ResetPrivacy_G2

| Definition | int ResetPrivacy_G2(ref byte ComAddr,byte[] Password, ref int errorcode); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Unlock read protection of a tag. Only one tag is allowed to be place in the antenna effective area. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | Password | byte[] | [in] | 4 bytes, the access password. |
|  | ErrorCode | ref int | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.4.4	CheckPrivacy_G2

| Definition | int CheckPrivacy_G2(ref byte ComAddr, ref byte readpro, ref int errorcode); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is NOT ABLE TO identify if a specific tag supports the read protection function, the command ONLY inquire the read protection status of the tag. For tags with no read protection function, the default status will be unlocked. Only one tag is allowed to be place in the antenna effective area. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | readpro | ref byte | [in] | 1 byte. 0x00 – read protection is disabled for the tag 0x01 – read protection is enabled for the tag Note For tags with no read protection function, the default status will be unlocked. |
|  | ErrorCode | ref int | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.4.5	EASConfigure_G2

| Definition | int EASConfigure_G2(ref byte ComAddr, byte[] EPC, byte ENum, byte[] Password, byte EAS, byte MaskMem,byte[] MaskAdr, byte MaskLen, byte[] MaskData, ref int errorcode); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify or reset EAS status. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | byte[] | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | byte | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Password | byte[] | [in] | 4 bytes, the access password.。 |
|  | EAS | byte | [in] | 1 byte. 0x00 – EAS alert is disabled; 0x01 – EAS alert is enabled. |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte[] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte[] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | ref int | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.4.6	EASAlarm_G2

| Definition | int EASAlarm_G2(ref byte ComAddr,ref int errorcode); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Detect EAS alert. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | ErrorCode | ref int | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

## 3.5	18000-6B Commands

3.5.1	InventorySingle_6B

| Definition | int InventorySingle_6B(ref byte ConAddr, ref byte ant,byte[] ID_6B); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Inquire a single tag. If multiple tags are placed within the effective field, reader may fail to inquire any tag. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | Ant | ref byte | [out] | 1 byte. For example, 0x04 is 0000 0100 in binary, indicates Antenna 3 had inquired this specific tag. For 0x08, it is 0000 1000 in binary, indicates Antenna 4 had inquired tag information. |
|  | ID_6B | byte[] | [out] | 10 bytes, the 1st byte is the length of UID, 0x08. The 2nd ~ 9th byte is the tag UID number, least-significant byte first. The 10th byte is RSSI. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.5.2	InventoryMultiple_6B

| Definition | int InventoryMultiple_6B(ref byte ConAddr, byte Condition,byte StartAddress,byte mask,byte[] ConditionContent, ref byte ant, byte[] ID_6B, ref int Cardnum); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Inquire tags according to specified condition. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | Condition | byte | [in] | the condition for inventory; 0x00 – equal to the reference; 0x01 – no equal to the reference; 0x02 – larger than the reference 0x03 – smaller than the reference |
|  | StartAddress | byte | [in] | 1 byte, entry address of the target. |
|  | mask | byte | [in] | 1 byte, specify the data for comparison. Every bit in mask is corresponding to a byte in ConditionContent. The most-significant bit (bit7) corresponds to the 1st byte in ConditionContent (from left to right). Consequently, the least- significant bit (bit0) corresponds to the last byte in ConditionContent (from left to right). ConditionContent is the reference data for comparison with data in tag starts from the location stated in StartAddress. The 1st byte of ConditionContent will be compare with the data store in Address, the last byte of ConditionContent will be compare with the data store in Address+7. Set the byte corresponding bit of Mask to 1, reader will perform comparison on that particular byte. Set the bit to 0, no comparison will be performed on that particular byte. |
|  | ConditionContent | byte[] | [in] | 8 bytes, the value of reference. |
|  | Ant | ref byte | [out] | 1 byte. For example, 0x04 is 0000 0100 in binary, indicates Antenna 3 had inquired this specific tag. For 0x08, it is 0000 1000 in binary, indicates Antenna 4 had inquired tag information. |
|  | ID_6B | byte[] | [out] | Cardnum*10 bytes. UID of the tag. The length of every UID data block is 10 bytes. The 1st byte is the length value of UID number, i.e. 0x08. The 2nd ~ 9th bytes are the tag UID number. The 10th byte is the RSSI. |
|  | Cardnum | ref int | [in] | 1 byte, the total amount of tag inquired during the current inventory. |
| Return value (int) | 0x15: inventory completed, tag data will be delivered; 0x16: inventory timeout, operation is aborted, inquired tag data will be delivered. 0xF8: error occurred in antenna check, the target antenna might be disconnected. For description on other error, refer to reader error code table. |  |  |  |
| Sample code | N/A |  |  |  |

3.5.3	ReadData_6B

| Definition | int ReadData_6B(ref byte ConAddr, byte[] ID_6B, byte StartAddress, byte Num, byte[] Data, ref int errorcode); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Read multiple bytes from a specific address. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | ID_6B | byte[] | [in] | 8 bytes, UID of the target tag. |
|  | StartAddress | byte | [in] | 1 byte, the entry address of target. The valid value range of StartAddress is 0 ~ 223. Reader will return a parameter error status in the response frame if incorrect address is delivered. |
|  | Num | byte | [in] | 1 byte, the amount of words to be read in this operation. The valid value range of Num is 1 ~ 32. If the sum of StartAddress and Num is larger than 224 or Num is equal to 0 or larger than 32, reader will return a parameter error status in the response frame. |
|  | Data | byte[] | [out] | Inquired tag data, with a length stated in Num. |
|  | Errorcode | ref int | [out] | 1 byte, reserved. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.5.4	WriteData_6B

| Definition | int WriteData_6B(ref byte ConAddr, byte[] ID_6B, byte StartAddress, byte[] Writedata, byte Writedatalen, ref int writtenbyte, ref int errorcode); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Write multiple bytes to a specific tag. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | ID_6B | byte[] | [in] | 8 bytes, UID of the target tag. |
|  | StartAddress | byte | [in] | 1 byte, the entry address of target. The valid value range of StartAddress is 0 ~ 223. Reader will return a parameter error status in the response frame if incorrect address is delivered. |
|  | Writedata | byte[] | [in] | Data to be written to tag, with a length stated in Writedatalen. |
|  | Writedatalen | byte | [in] | 1 byte, the amount of words to be written in this operation. |
|  | writtenbyte | byte[] | [out] | 1 byte, reserved. |
|  | Errorcode | ref int | [out] | 1 byte, reserved. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.5.5	CheckLock_6B

| Definition | int CheckLock_6B(ref byte ConAddr, byte[] ID_6B, byte Address, ref byte ReLockState, ref int errorcode); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain the lock status of a specific byte. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | ID_6B | byte[] | [in] | 8 bytes, UID of the target tag. |
|  | StartAddress | byte | [in] | 1 byte, the entry address of target. The valid value range of StartAddress is 0 ~ 223. Reader will return a parameter error status in the response frame if incorrect address is delivered. |
|  | ReLockState | ref byte | [out] | 1 byte. 0x00: the byte is unlocked; 0x01: the byte is already locked. |
|  | Errorcode | ref int | [out] | 1 byte, reserved. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.5.6	Lock_6B

| Definition | int Lock_6B(ref byte ConAddr, byte[] ID_6B, byte Address, ref int errorcode); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Lock a specific byte in a tag. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | ID_6B | byte[] | [in] | 8 bytes, UID of the target tag. |
|  | StartAddress | byte | [in] | 1 byte, the entry address of target. The valid value range of StartAddress is 0 ~ 223. Reader will return a parameter error status in the response frame if incorrect address is delivered. |
|  | errorcode | ref int | [in] | 1 byte, reserved. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

## 3.6	Reader customised commands

3.6.1	GetReaderInformation

| Definition | int GetReaderInformation(ref byte ComAddr, byte[] VersionInfo, ref byte ReaderType, ref byte TrType, ref byte dmaxfre, ref byte dminfre, ref byte powerdBm, ref byte ScanTime,ref byte Ant, ref byte BeepEn, ref byte OutputRep, ref byte CheckAnt); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain reader information, including reader address (ComAddr), firmware version (VersionInfo) and other reader information. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands.。 |
|  | VersionInfo | byte[] | [out] | 2 bytes, firmware version. The 1st byte: the main version number; The 2nd byte: the subversion number. |
|  | ReaderType | ref byte | [out] | 1 byte. Reader Type code. |
|  | TrType | ref byte | [out] | 1 byte, reader supported protocol. |
|  | dmaxfre | ref byte | [out] | 1 byte. bit7 ~ bit6: frequency band configuration; bit5 ~ bit0: current maximum frequency point. |
|  | dminfre | ref byte | [out] | 1 byte. bit7 ~ bit6: frequency band configuration; bit5 ~ bit0: current minimum frequency point. |
|  | powerdBm | ref byte | [out] | 1 byte, Output RF power. The range is 0 ~ 33 and the unit is dBm. |
|  | ScanTime | ref byte | [out] | 1 byte, inventory time. Reader will respond to the inventory command delivered from host within this specific inventory time. |
|  | Ant | ref byte | [out] | 1 byte, antenna configuration. |
|  | BeepEn | ref byte | [out] | 1 byte, buzzer status information. |
|  | OutputRep | ref byte | [out] | 1 byte, GPIO status for notification. |
|  | CheckAnt | ref byte | [out] | 1 byte, Antenna check configuration. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

Frequency band configuration table

| MaxFre (bit7) | MaxFre (bit6) | MinFre (bit7) | MinFre (bit6) | FreqBand |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | Chinese band2 |
| 0 | 0 | 1 | 0 | US band |
| 0 | 0 | 1 | 1 | Korean band |
| 0 | 1 | 0 | 0 | EU band |
| 0 | 1 | 1 | 0 | Ukraine band |
| 1 | 0 | 0 | 0 | Chinese band1 |
| 1 | 1 | 0 | 0 | US band3 |

3.6.2	SetAddress

| Definition | int SetAddress(ref byte ComAddr,byte ComAddrData); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Reader will apply the user provided value as its reader address and write this value to EEPROM for preservation purpose. Default setting of reader address is 0x00, the valid value of the setting is 0x00 ~ 0x0FE. Reader address will be reset to 0x00, if the provided address value is 0xFF. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | ComAddrData | BYTE | [in] | 1 byte, the new address setting. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.3	SetInventoryScanTime

| Definition | int SetInventoryScanTime(ref byte ComAddr, byte ScanTime); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify maximum operation time for inventory command. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | ScanTime | byte | [in] | 1 byte, operation time for inventory. The acceptable range of Scantime is 0 ~ 255, corresponding to (0 ~ 255)*100ms. For Scantime = 0, operation time is not limited until a whole reading cycle is completed. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.4	SetRfPower

| Definition | int SetRfPower(ref byte ComAddr,byte powerDbm); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify reader RF power |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | PowerDbm | byte | [in] | 1 byte, reader RF power. The range is 0 ~ 33 and the unit is dBm. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

| Definition | int SetRfPower(ref byte ComAddr,byte[] powerDbm); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify reader RF power |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | PowerDbm | byte[] | [in] | 4/8/16 bytes, reader power, range 0~33dBm. The highest bit Bit7 is 0, set to save when power off, 1 to not save when power off, set the power according to the antenna, such as passing 8 bytes, it means that the power is set in sequence for the 8 antennas of the 8-port device |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.5	SetRegion

| Definition | int SetRegion(BYTE *address, BYTE dmaxfre, BYTE dminfre, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Select maximum and minimum frequency. Maximum frequency must be larger or equal the minimum frequency. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | dmaxfre | byte | [in] | 1 byte. bit7 ~ bit6: frequency band configuration; bit5 ~ bit0: maximum frequency point. |
|  | dminfre | byte | [in] | 1 byte. bit7 ~ bit6: frequency band configuration; bit5 ~ bit0: minimum frequency point. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table) |  |  |  |
| Sample code | N/A |  |  |  |

Frequency band configuration table

| MaxFre (bit7) | MaxFre (bit6) | MinFre (bit7) | MinFre (bit6) | FreqBand |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | Chinese band2 |
| 0 | 0 | 1 | 0 | US band |
| 0 | 0 | 1 | 1 | Korean band |
| 0 | 1 | 0 | 0 | EU band |
| 0 | 1 | 1 | 0 | Ukraine band |
| 1 | 0 | 0 | 0 | Chinese band1 |
| 1 | 1 | 0 | 0 | US band3 |

Formulae for different frequency bands:

Chinese band2: Fs = 920.125 + N * 0.25 (MHz) where N∈ [0, 19]. US band: Fs = 902.75 + N * 0.5 (MHz) where N∈ [0, 49]. Korean band: Fs = 917.1 + N * 0.2 (MHz) where N∈ [0, 31]. EU band: Fs = 865.1 + N*0.2(MHz) where N∈ [0, 14]. Ukraine band: Fs = 868.0 + N*0.1(MHz) where N∈ [0, 6]. Chinese band1: Fs = 840.125 + N * 0.25 (MHz) where N∈ [0, 19]. US band3: Fs = 902 + N * 0.5 (MHz) where N∈ [0, 52].

3.6.6	SetBaudRate

| Definition | int SetBaudRate(ref byte ComAddr,byte baud); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify serial baud rate. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | baud | byte | [in] | 1 byte, serial baud rate. 0 – 9600bps; 1 – 19200bps; 2 – 38400bps; 5 – 57600bps; 6 – 115200bps. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.7	BuzzerAndLEDControl

| Definition | int BuzzerAndLEDControl(ref byte ComAddr, byte AvtiveTime, byte SilentTime, byte Times); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Control the TAG LED and buzzer to perform specific action. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | AvtiveTime | byte | [in] | 1 byte, TAG LED and buzzer switch ON time (ActiveT*50ms). The range of ActiveT is 0 ~ 255. Default setting is 0. |
|  | SilentTime | byte | [in] | 1 byte, TAG LED and buzzer switch OFF time (SilentT*50ms). The range of SilentT is 0 ~ 255. Default setting is 0. |
|  | Times | byte | [in] | TAG LED and buzzer action time, The range of Times is 0 ~ 255. Default setting is 0. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.8	SetAntennaMultiplexing

| Definition | public int SetAntennaMultiplexing(ref byte ComAddr, byte Ant); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify antenna configuration of 4 ports reader. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | Ant | byte | [in] | 1 byte, antenna configuration information. 1 antenna is the MINIMUM requirement for operation. bit0: antenna 1 configuration bit. 0 – disable antenna 1; 1 – enable antenna 1. bit1: antenna 2 configuration bit. 0 – disable antenna 2; 1 – enable antenna 2. bit2: antenna 3 configuration bit. 0 – disable antenna 3; 1 – enable antenna 3. bit3: antenna 4 configuration bit. 0 – disable antenna 4; 1 – enable antenna 4. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.9	SetAntennaMultiplexing

| Definition | public int SetAntennaMultiplexing(ref byte ComAddr, byte SetOnce, byte AntCfg1, byte AntCfg2); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify antenna configuration of 8 or 12 or 16 ports reader. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | SetOnce | byte | [in] | 1 byte, configuration preservation status during power off. 0x00 – configuration preserved during reader power off; 0x01 – configuration is not preserved. |
|  | AntCfg1 | byte | [in] | 1 byte, antenna configuration information. bit0: antenna 9 configuration bit. 0 – disable antenna 9; 1 – enable antenna 9. bit1: antenna 10 configuration bit. 0 – disable antenna 10; 1 – enable antenna 10. bit2: antenna 11 configuration bit. 0 – disable antenna 11; 1 – enable antenna 11. bit3: antenna 12 configuration bit. 0 – disable antenna 12; 1 – enable antenna 12. bit4: antenna 13 configuration bit. 0 – disable antenna 13; 1 – enable antenna 13. bit5: antenna 14 configuration bit. 0 – disable antenna 14; 1 – enable antenna 14. bit6: antenna 15 configuration bit. 0 – disable antenna 15; 1 – enable antenna 15. bit7: antenna 16 configuration bit. 0 – disable antenna 16; 1 – enable antenna 16. |
|  | AntCfg2 | byte | [in] | 1 byte, antenna configuration information. bit0: antenna 1 configuration bit. 0 – disable antenna 1; 1 – enable antenna 1. bit1: antenna 2 configuration bit. 0 – disable antenna 2; 1 – enable antenna 2. bit2: antenna 3 configuration bit. 0 – disable antenna 3; 1 – enable antenna 3. bit3: antenna 4 configuration bit. 0 – disable antenna 4; 1 – enable antenna 4. bit4: antenna 5 configuration bit. 0 – disable antenna 5; 1 – enable antenna 5. bit5: antenna 6 configuration bit. 0 – disable antenna 6; 1 – enable antenna 6. bit6: antenna 7 configuration bit. 0 – disable antenna 7; 1 – enable antenna 7. bit7: antenna 8 configuration bit. 0 – disable antenna 8; 1 – enable antenna 8. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.10	SetBeepNotification

| Definition | int SetBeepNotification(ref byte ComAddr, byte BeepEn); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Enable/disable buzzer. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | BeepEn | byte | [in] | 1 byte. 0x00 – disable buzzer; 0x01 – enable buzzer. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.11	SetReal_timeClock

| Definition | int SetReal_timeClock(ref byte ComAddr, byte[] paramer) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Adjust real time clock. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | paramer | byte[] | [in] | 6 bytes, including Year, Month, Day, Hour, Minute and Second. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.12	GetTime

| Definition | int GetTime(ref byte ComAddr,byte[] paramer); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Get current time from real time clock. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | paramer | byte[] | [out] | 6 bytes, including Year, Month, Day, Hour, Minute and Second. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.13	SetRelay

| Definition | int SetRelay(ref byte ComAddr,byte RelayTime); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Control the state of the built-in relay. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | RelayTime | byte | [in] | Configuration of the relay pickup time, the valid range of the parameter is 1 ~ 255, corresponds to (1 ~ 255)*50ms |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.14	GetGPIOStatus

| Definition | int GetGPIOStatus(ref byte ComAddr, ref byte OutputPin); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain the input and output states of GPIO. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | OutputPin | ref byte | [out] | 1 byte, bit0 ~ bit3 represent IN1 to IN4 and bit4 ~ bit7 represent Out1 to Out4 respectively. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.15	SetGPIO

| Definition | int SetGPIO(ref byte ComAddr,byte OutputPin); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Control the output of GPIO. The default output is high TTL level. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | OutputPin | byte | [in] | 1 byte, the output state of GPIO (pin Out1 to Out4). Bit0 ~ bit3 control Out1 to Out4 respectively. Bit4 ~ bit7 are reserved. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.16	SetNotificationPulseOutput

| Definition | int SetNotificationPulseOutput(ref byte ComAddr, byte OutputRep); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Enable / disable a 2ms low level notification pulse on the associated port for every tag detection. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | OutputRep | byte | [in] | 1 byte. bit0: associated with antenna 1 and Out1 pin. 1 – Out1 will output a 2ms low level notification pulse for every tag detection from antenna 1; 0 – disable notification signal. bit1: associated with antenna 2 and Out2 pin. 1 – Out2 will output a 2ms low level notification pulse for every tag detection from antenna 2; 0 – disable notification signal. bit2: associated with antenna 3 and Out3 pin. 1 – Out3 will output a 2ms low level notification pulse for every tag detection from antenna 3; 0 – disable notification signal. bit3: associated with antenna 4 and Out4 pin. 1 – Out4 will output a 2ms low level notification pulse for every tag detection from antenna 4; 0 – disable notification signal. All other values are reserved, default is 0. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.17	SetTIDParameter

| Definition | int SetTIDParameter(ref byte ComAddr,byte AdrTID,byte LenTID); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify the parameter for TID inventory |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | AdrTID | byte | [in] | 1 byte, entry address of TID memory inventory. |
|  | LenTID | byte | [in] | 1 byte, data length for TID inventory operation, the valid range of LenTID is 0 ~ 15. Reader will return a parameter error status in the response frame if incorrect value is delivered in this field. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.18	SetQS

| Definition | int SetQS(ref byte ConAddr,byte Qvalue, byte Session); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify parameters for multiple-tag inventory in auto-mode. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | Qvalue | byte | [in] | 1 byte, the Q-value of inventory. The valid range of Q-value is 0 ~ 15. |
|  | Session | byte | [in] | 1 byte, the Session-value of inventory. 0x00 – apply S0 as Session value; 0x01 – apply S1 as Session value; 0x02 – apply S2 as Session value; 0x03 – apply S3 as Session value. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.19	GetQS

| Definition | int GetQS(ref byte ConAddr, ref byte Qvalue, ref byte Session); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain configuration for multiple-tag inventory in auto-mode. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | Qvalue | ref byte | [out] | 1 byte, the Q-value of inventory. |
|  | Session | ref byte | [out] | 1 byte, the Session-value of inventory. 0x00 – applied S0 as Session value; 0x01 – applied S1 as Session value; 0x02 – applied S2 as Session value; 0x03 – applied S3 as Session value. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.20	GetSeriaNo

| Definition | int GetSeriaNo(ref byte ConAddr, byte[] SeriaNo); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain 4 bytes reader unique serial number. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | SeriaNo | byte[] | [out] | 4 bytes, the unique serial number of reader. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.21	SetCheckAnt

| Definition | int SetCheckAnt(ref byte ComAddr, byte CheckAnt); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Enable antenna check during tag reading/writing operations. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | CheckAnt | byte | [in] | 1 byte, antenna check switch. 0x00 – disable antenna check; 0x01 – enable antenna check. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.22	SetWorkMode

| Definition | int SetWorkMode(ref byte ComAddr, byte Read_mode); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Set reader working mode (only supported by UHFReader288 and UHFReader288ADV). |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | Read_mode | byte | [in] | 1 byte, working mode. 0x00 – answer mode; 0x01 – auto mode; 0x02 – trigger mode(low level active); 0x03 – trigger mode(high level active); 0x06 – real time mode. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.23	GetSystemParameter

| Definition | int GetSystemParameter(ref byte ComAddr,ref byte Read_mode,ref byte Accuracy, ref byte RepCondition, ref byte RepPauseTime, ref byte ReadPauseTim, ref byte TagProtocol, ref byte MaskMem, byte[] MaskAdr, ref byte MaskLen, byte[] MaskData, ref byte TriggerTime, ref byte AdrTID, ref byte LenTID); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain auto mode parameters (only supported by UHFReader288, UHFReader288ADV). |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | Read_mode | ref byte | [out] | 1 byte, working mode information. |
|  | Accuracy | ref byte | [out] | 1 byte, accuracy parameter of EAS check in auto-mode. |
|  | RepCondition | ref byte | [out] | 1 byte, parameter of answering form in auto-mode. |
|  | RepPauseTime | ref byte | [out] | 1 byte, parameter of answering form in auto-mode. |
|  | ReadPauseTim | ref byte | [out] | 1 byte, time break between two inventories in auto-mode. |
|  | TagProtocol | ref byte | [out] | 1 byte, tag type for auto-mode inventory. |
|  | MaskMem | ref byte | [out] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte[] | [out] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | ref byte | [out] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte[] | [out] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | TriggerTime | ref byte | [out] | 1 byte, the effective trigger time in trigger mode. |
|  | AdrTID | ref byte | [out] | 1 byte, initial byte address for TID memory read operation. |
|  | LenTID | ref byte | [out] | 1 byte, data length for the TID read operation. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.24	ReadActiveModeData

| Definition | int ReadActiveModeData(byte[] ScanModeData, ref int ValidDatalength); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain reader uploaded data in auto-mode or real time mode. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ScanModeData | byte[] | [out] | Reader uploaded raw data, with a length stated in Datalength. Refer to user manual for detail format. |
|  | ValidDatalength | ref int | [out] | Byte length of LoadData. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.25	SetEASSensitivity

| Definition | int SetEASSensitivity(ref byte ComAddr,byte Accuracy); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Read data from a specified data block, and obtain the block EAS parameter byte. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | Accuracy | byte | [in] | 1 byte. bit7: 1 – close relay for 100ms once EAS is detected in auto-mode; 0 – relay remains in current state. bit6 ~ bit0: the valid range is 0 ~ 8, lower value represent higher sensitivity but lower accuracy. Default setting is 8, i.e. highest accuracy and lowest sensitivity. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.26	SetMask

| Definition | int SetMask(ref byte ComAddr, byte MaskMem, byte[] MaskAdr, byte MaskLen, byte[] MaskData); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify the parameters of G2 tag inventory in auto-mode. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte[] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte[] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0.。 |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.27	SetResponsePamametersofAuto_runningMode

| Definition | int SetResponsePamametersofAuto_runningMode(BYTE *ComAddr, BYTE RepCondition, BYTE RepPauseTime,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify reader answering form in auto-mode. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | RepCondition | byte | [in] | 1 byte, options for answering form in auto-mode. 0x00 – notify upon command. Reader will only produce tag information report upon notification command reception; 0x01 – notify over time. Reader will produce tag information report in every RepPauseTime time interval; 0x02 – notify upon new tag. Reader will produce tag information report and store the information once a new tag is inquired; 0x03 – notify upon tag information removal. Reader will produce tag information report upon tag information deletion due to lack of memory. 0x04 – notify upon tag information changes. Reader will produce tag information report upon tag information adding or removal in the memory. All other values are reserved, reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | RepPauseTime | byte | [in] | 1 byte, the time break between two tag information reports, in notify over time form. The range of RepPauseTime is (0 ~ 255)*1s. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.28	SetInventoryInterval

| Definition | int SetInventoryInterval(ref byte ComAddr,byte ReadPauseTim); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify the inventory time break in auto-mode. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | ReadPauseTim | byte | [in] | 1 byte, time break between 2 real time inventories. 0x00 – 10ms; 0x01 – 20ms; 0x02 – 30ms; 0x03 – 50ms; 0x04 – 100ms. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.29	SelectTagType

| Definition | int SelectTagType(ref byte ComAddr, byte Protocol); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify the inventory tag type in auto-mode. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | TagProtocol | byte | [in] | 1 byte. bit7: tag protocol type. 0 – set inventory on G2(18000-6C) tag; for TagProtocol = 0x00, only inquired EPC of G2 tags; for TagProtocol = 0x02, only detects EAS; for TagProtocal = 0x10, only inquired TID of G2 tags. 1 – for TagProtocol = 0x80, set inventory on 18000-6B tag. All other value reserved. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.30	GetTagBufferInfo

| Definition | int GetTagBufferInfo(ref byte ComAddr,byte[] Data, ref int dataLength); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain all the tag information from reader memory. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | Data | byte[] | [out] | Reader stored data, with a length stated in Datalength. |
|  | dataLength | ref int | [out] | Byte length of Data. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.31	ClearTagBuffer

| Definition | int ClearTagBuffer(ref byte ComAddr); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Clear all the stored tag data from the memory buffer |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.32	GetModuleVersion

| Definition | int GetModuleVersion(ref byte ConAddr,byte[] Version); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain reader internal R2000 module firmware version. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | Version | byte[] | [out] | 2 bytes, R2000 module version number. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.33	SetFlashRom

| Definition | int SetFlashRom(ref byte ConAddr); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Set the reader to R2000 update mode. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.34	WriteRfPower

| Definition | int WriteRfPower(ref byte ComAddr,byte powerDbm); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify RF power configuration separately for write operations |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | PowerDbm | byte | [in] | 1 byte, separate reader RF power for write operation. The range is 0 ~ 33 and the unit is dBm. bit7 = 0: disable; bit7 = 1: enable. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.35	ReadRfPower

| Definition | int ReadRfPower(ref byte ComAddr, ref byte powerDbm); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain RF power configuration for write operations |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | PowerDbm | ref byte | [out] | 1 byte, separate reader RF power for write operation. The range is 0 ~ 33 and the unit is dBm. bit7 = 0: disabled; bit7 = 1: enabled. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.36	RetryTimes

| Definition | int RetryTimes(ref byte ComAddr, ref byte Times); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify or load the maximum operation retry time configuration |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | Times | ref byte | [out] | 1 byte. bit7 = 1: modify maximum write retry time; bit7 = 0: load maximum write retry time The valid range of configuration is 0 ~ 7. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.37	SetReadMode

| Definition | int SetReadMode(ref byte ComAddr,byte ReadMode); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Select reader working mode. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | ReadMode | byte | [in] | 1 byte. 0x00 – answer mode; 0x01 – real time inventory mode; 0x02 – trigger mode. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.38	SetReadParameter

| Definition | int SetReadParameter(ref byte ComAddr,byte[] Parameter, byte MaskMem,byte[] MaskAdr, byte MaskLen,byte[] MaskData, byte MaskFlag, byte AdrTID, byte LenTID, byte TIDFlag); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify the real time inventory associated parameters. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | Parameter | byte[] | [in] | Parameters definition 5 bytes. Parameter[0]: 1 byte, tag type definition for real time inventory. 0 – EPC C1G2 (ISO18000-6C) tags. 1 – ISO18000-6B tags. All other values are reserved, reader will return a parameter error status in the response frame if other value is delivered in this field. Parameter[1]: 1 byte, time break between 2 real time inventories. 0x00 – 10ms; 0x01 – 20ms; 0x02 – 30ms; 0x03 – 50ms; 0x04 – 100ms. Parameter[2]: 1 byte, tag filtering time. The valid value of this parameter is 0 ~ 255, corresponds to (0 ~ 255)*1s. In real time inventory, if reader detects a particular tag for more than 1 time, reader will only upload tag information of this tag once within the pre-defined filtering time. For FliterTime = 0, disable tag filtering function. Parameter[3]: 1 byte, the original Q-value of the EPC tag inventory, the setting of Q-value should follows the rule: 2Q ≈ total amount of tags within the effective field. The range of Q-value is 0 ~ 15, if other value is delivered in this field, reader will return a parameter error status in the response frame. Parameter[4]: 1 byte, the Session-value of the EPC tag inventory. 0x00 – apply S0 as Session value; 0x01 – apply S1 as Session value; 0x02 – apply S2 as Session value; 0x03 – apply S3 as Session value; 0xff – apply reader smart configuration (only valid in EPC inventory). |
|  | MaskMem | byte | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | byte [] | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | byte | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | byte[] | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | AdrTID | byte | [in] | 1 byte, entry address of TID memory inventory. |
|  | LenTID | byte | [in] | 1 byte, data length for TID inventory operation, the valid range of LenTID is 0 ~ 15. |
|  | TIDFlag | byte | [in] | 1 byte, inventory purpose indicator. 0 – EPC inventory; 1 – TID inventory. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.39	GetReadParameter

| Definition | int GetReadParameter(ref byte ComAddr, byte[] Parameter); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain the real time inventory associated parameters. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | Parameter | byte[] | [out] | Parameters definition ReadMode: 1 byte, the current working mode of reader. TagProtocol: 1 byte, tag type definition of real time inventory. ReadPauseTime: 1 byte, time break between 2 real time inventories. FliterTime: 1 byte, tag filtering time of real time inventory. QValue: 1 byte, the original Q-value of real time mode for EPC inventory. Session: 1 byte, the Session-value of real time mode for EPC inventory. MaskMem, MaskAdr, MaskLen, MaskData: mask condition for EPC C1G2 tag inventory in real time mode. MaskMem and MaskLen are 1 byte long respectively. MaskAdr is 2 bytes long, most-significant byte first. MaskData has constant length of 32 bytes, fill the over MaskLen value content with 0. AdrTID: entry address of TID memory inventory. LenTID: data length for TID inventory operation. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.40	SetAntennaPower

| Definition | int SetAntennaPower(ref byte ComAddr,byte[] powerDbm,int length); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Set the individual power of each antenna of the reader. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | powerDbm | byte[] | [in] | Reader power, byte0-byte3 represent the power of antenna 1-antenna 4 respectively. Bit0~Bit6 of each byte, the antenna power parameter to be set, the range is 0~33, when the value is 30, the output power is about 1W. Bit7: Indicates whether the current configuration is saved after power-off. Bit7=0 save when power off; Bit7=1 not save when power off. |
|  | length | int | [in] | The number of antenna ports to set, the values are 4, 8, and 16. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.41	GetAntennaPower

| Definition | int GetAntennaPower(ref byte ComAddr,byte[] powerDbm,ref int length); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Get the individual power of each antenna of the reader. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | powerDbm | byte[] | [out] | length bytes, the power of the reader, the high order is first, the range is 0~30, and the output power is about 1W when the value is 30. |
|  | length | ref int | [in] | Return the size of powerDbm, the value is 4,8,16 |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.42	SetCfgParameter

| Definition | int SetCfgParameter(ref byte ComAddr,byte opt,byte cfgNum,byte[] data,int len); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to configure extended parameters, See user manual for details. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | opt | byte | [in] | 0-save;1-not save; |
|  | cfgNum | byte | [in] | Used to specify the parameter sequence number to be set in the command data block |
|  | data | byte[] | [in] | Set the parameter content, see the reader parameter configuration table for details. |
|  | len | int | [in] | Parameter content length. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.43	GetCfgParameter

| Definition | int GetCfgParameter (ref byte ComAddr, byte cfgNum,byte[] data,ref int len); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to read extended parameters. See user manual for details |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | ref byte | [in/out] | Reader address, obtained from open port commands. |
|  | cfgNum | byte | [in] | Used to specify the parameter sequence number to be get in the command data block |
|  | data | byte[] | [out] | Read the parameter content, see the reader parameter configuration table for details |
|  | len | ref int | [out] | Parameter content length. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

# Appendix 1: Reader Error code table

| Error code | Description |
| --- | --- |
| 0x00 | API is called successfully. |
| 0x01 | Tag inventory is completed successfully and reader is able to deliver data response within the predefined inventory time. |
| 0x02 | Inventory timeout. |
| 0x05 | Access password error. |
| 0x09 | Kill password error. |
| 0x0A | All-zero tag killing password is invalid. |
| 0x0B | Command is not support by the tag |
| 0x0C | All-zero tag access password is invalid for such command. |
| 0x0D | Fail to setup read protection for a protection enabled tag. |
| 0x0E | Fail to unlock a protection disabled tag. |
| 0x10 | Some bytes stored in the tag are locked. |
| 0x11 | Lock operation failed. |
| 0x12 | Already locked, lock operation failed. |
| 0x13 | Fail to store the value of some preserved parameters. Configuration will still valid before reader shut down. |
| 0x14 | Modification failed. |
| 0x15 | Response within the predefined inventory time. |
| 0x17 | Further data is waiting to be delivered. |
| 0x18 | Reader memory is full. |
| 0x19 | All-zero access password is invalid for such command or command is not supported by the tag. |
| 0xF8 | Error detected in antenna check. |
| 0xF9 | Operation failed. |
| 0xFA | Tag is detected, but fails to complete operation due to poor communication. |
| 0xFB | No tag is detected. |
| 0xFC | Error code returned from tags. |
| 0xFD | Command length error. |
| 0xFE | Illegal command. |
| 0xFF | Parameter error. |
| 0x30 | Communication error. |
| 0x33 | Reader is busy, operation in process. |
| 0x35 | Port is already opened. |

# Appendix 2: Tag ErrorCode table

| ErrorCode | Description |
| --- | --- |
| 0x00 | Other errors, all other Errorcode non-specified error. |
| 0x03 | Memory overload, location is not found or unsupported PC value. |
| 0x04 | Memory is being temporarily / permanently locked, unable to perform write operation. |
| 0x0B | Unable to perform write operation due to insufficient power supply to tag. |
| 0x0F | Undefined or tag unsupported errors. |