CReader_Uhf.jar Manual

1. programming interface	2

2. parameter interface	3

3. callback interface	3

4. Interface details	4

4.1 Open/Close RFID	4

4.1.1 CReader	4

4.1.2 Connect	4

4.1.3 Disconnect	4

4.2 18000-6C	5

4.2.1 StartRead	5

4.2.2 StopRead	5

4.2.3 ReadDataByEPC	5

4.2.4 ReadDataByTID	6

4.2.5 WriteDataByEPC	6

4.2.6 WriteDataByTID	7

4.2.7 WriteEPCByTID	7

4.2.8 Lock	8

4.2.9 Kill	9

4.2.10 ReadData_G2	9

4.2.11 WriteData_G2	10

4.2.12 Inventory_G2	11

4.2.13 FindEPC	12

4.2.14 LedOn_kx2005x	12

4.3 Custom command	13

4.3.1 SetCallBack	13

4.3.2 SetInventoryParameter	13

4.3.3 GetInventoryParameter	13

4.3.4 GetUHFInformation	14

4.3.5 SetRfPower	14

4.3.6 SetAntenna	15

4.3.7 SetGPIO	15

4.3.8 GetGPIOStatus	16

4.3.9 SetRegion	16

4.3.10 SetBeepNotification	17

4.3.11 SetRfPowerByAnt	17

4.3.12 GetRfPowerByAnt	17

4.3.13 SetRelay	18

4.3.14 GetDeviceID	18

4.3.15 MeasureReturnLoss	18

4.3.16 SetWritePower	19

4.3.17 GetWritePower	19

4.3.18 SetCheckAnt	19

4.4 Return value error code table	19

- 1. programming interface

Import CReader.jar package and *.so file

Declare: import com.rfid.trans2000.UHFLib;

Interface preview：

public UHFLib(int type,String DevName);

public int Connect(String ComPort,int BaudRate);

public int DisConnect();

public void SetInventoryParameter(ReaderParameter param);

public ReaderParameter GetInventoryPatameter();

public void SetCallBack(TagCallback callback);

public int StartRead();

public void StopRead();

public int GetUHFInformation(byte Version[],byte Power[],byte band[],byte MaxFre[],byte MinFre[],byte BeepEn[],int Ant[]);

public int SetRfPower(int Power);

public int SetRegion(int band,int maxfre,int minfre);

public int SetAntenna(int SetOnce,int AntCfg);

public int SetBeepNotification(int BeepEn);

public int SetRfPowerByAnt(byte Power[]);

public int GetRfPowerByAnt(byte Power[]);

public int SetRelay(int RelayTime);

public int SetGPIO(byte GPIO);

public int GetGPIOStatus(byte OutputPin[]);

public String GetDeviceID();

public int MeasureReturnLoss(byte TestFreq[],byte Ant,byte ReturnLoss[]);

public int SetWritePower(byte WritePower);

public int GetWritePower(byte WritePower[]);

public int SetCheckAnt(byte CheckAnt);

public String ReadDataByEPC(String EPC,byte Mem,byte WordPtr,byte Num,

String Password);

public String ReadDataByTID(String TID,byte Mem,byte WordPtr,

byte Num, String Password);

public int WriteDataByEPC(String EPC,byte Mem,byte WordPtr, String Password,

String wdata);

public int WriteDataByTID(String TID,byte Mem,byte WordPtr, String Password,

String wdata);

public int WriteEPCByTID(String TID,String EPC, String Password);

public int WriteEPC(String EPC, String Password);

public int Lock(String EPC,byte select,byte setprotect,String Password);

public int Kill(String EPC,String Killword);

public int Inventory(byte session,byte qvalue,byte tidAddr,byte tidLen,byte antenna,byte target,byte scantime,List<ReadTag> tagList);

public int FindEPC(String EPC,int antenna,List<ReadTag> tagList);

public int LedOn_kx2005x(String EPCStr,byte CarrierTime, byte Password[]);

- 2. parameter interface

public class ReaderParameter {

public byte ComAddr;

public int IvtType;//查询类型;0-EPC;1-TID;2-混合;3-GJB;4 GJB混合

public int Memory;

public String Password;

public int TidPtr;

public int TidLen;

public int Session;0~3:S0~S3;255-AUTO1;254-AUTO2;253-AUTO3.

public int QValue;

public int ScanTime;

public int Target;

public int reTryCount;

public int Antenna;

public int MaxAntennaNum;

public int Algo;

public byte MaskMem;

public byte[] MaskAddr = new byte[2];

public byte MaskLen;

public byte[] MaskData = new byte[32];

}

- 3. callback interface

public interface TagCallback {

public void tagCallback(ReadTag tag);

public int tagCallbackFailed(int reason);

public void ReadOver();

}

public class ReadTag {

public String epcId;// EPC or TID

public String memId;//memory

public int rssi;

public int phase_begin;

public int phase_end;

public int Freqkhz;

public int antId;//Antenna

public String DevName;//Device Name

}

- 4. Interface details

- 4.1 Open/Close RFID

#### 4.1.1 CReader

| Definition | public UHFLib(int type,String DevName) |  |  |
| --- | --- | --- | --- |
| Description | Create an object for connection management with the reader. |  |  |
| Parameters | Name | Type | Notes |
|  | type | int | Connection mode, 0-RS232 connection; 1-TCP connection. |
|  | DevName | String | custom name |
| Return value () |  |  |  |
| Sample code |  |  |  |

#### 4.1.2 Connect

| Definition | public int Connect(String Param1,int Param2); |  |  |
| --- | --- | --- | --- |
| Description | Connect with reader. |  |  |
| Parameters | Name | Type | Notes |
|  | Param1 | String | RS232: indicates the serial port name TCP: Indicates the IP address of the reader |
|  | Param2 | int | RS232:BaudRate TCP: socket port. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |
| Sample code |  |  |  |

#### 4.1.3 Disconnect

| Definition | int Disconnect(); |  |  |
| --- | --- | --- | --- |
| Description | Disconnect with reader, |  |  |
| Parameters | Name | Type | Notes |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |
| Sample code |  |  |  |

- 4.2 18000-6C

#### 4.2.1 StartRead

| Definition | Public int StartRead |  |  |
| --- | --- | --- | --- |
| Description | Start to read tag |  |  |
| Parameters | Name | Type | Notes |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |
| Sample code |  |  |  |

#### 4.2.2 StopRead

| Definition | Public void StopRead(); |  |  |
| --- | --- | --- | --- |
| Description | Stop to read tag |  |  |
| Parameters | Name | Type | Notes |
| Return value (void) |  |  |  |
| Sample code |  |  |  |

#### 4.2.3 ReadDataByEPC

| Definition | public String ReadDataByEPC(String EPCStr,byte Mem,byte WordPtr,byte Num, String Password); |  |  |
| --- | --- | --- | --- |
| Description | Read each memory area data through EPC mask |  |  |
| Parameters | Name | Type | Notes |
|  | EPCStr | String | The hexadecimal EPC number of the tag |
|  | Mem | byte | memory area to be read, 0- Password area, the first 2 words are the destruction password, the last 2 words are the access password 1- EPC area 2- TID area 3- User area |
|  | WordPtr | byte | The starting word address for reading |
|  | Num | byte | read word length |
|  | Password | String | Tag's hexadecimal access password, 4 bytes |
| Return value (String) | Success: tag data Failed: null; |  |  |
| Sample code |  |  |  |

#### 4.2.4 ReadDataByTID

| Definition | public String ReadDataByTID(String TIDStr,byte Mem,byte WordPtr,byte Num, String Password); |  |  |
| --- | --- | --- | --- |
| Description | Read each memory area data through TID mask |  |  |
| Parameters | Name | Type | Notes |
|  | TIDStr | String | The hexadecimal EPC number of the tag |
|  | Mem | byte | memory area to be read, 0- Password area, the first 2 words are the destruction password, the last 2 words are the access password 1- EPC area 2- TID area 3- User area |
|  | WordPtr | byte | The starting word address for reading |
|  | Num | byte | read word length |
|  | Password | String | Tag's hexadecimal access password, 4 bytes |
| Return value (String) | Success: tag data Failed: null; |  |  |
| Sample code |  |  |  |

#### 4.2.5 WriteDataByEPC

| Definition | public int WriteDataByEPC(String EPCStr,byte Mem,byte WordPtr,String Password,String wdata); |  |  |
| --- | --- | --- | --- |
| Description | Write data to each memory area through the EPC mask. |  |  |
| Parameters | Name | Type | Notes |
|  | EPCStr | String | The hexadecimal EPC number of the tag |
|  | Mem | byte | memory area to be write, 0- Password area, the first 2 words are the destruction password, the last 2 words are the access password 1- EPC area 2- TID area 3- User area |
|  | WordPtr | byte | Write start word address |
|  | Password | String | Tag's hexadecimal access password, 4 bytes |
|  | wdata | String | The hexadecimal string of the data to be written, the length must be an integer multiple of 4 |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

#### 4.2.6 WriteDataByTID

| Definition | public int WriteDataByTID(String TIDStr,byte Mem,byte WordPtr, String Password,String wdata); |  |  |
| --- | --- | --- | --- |
| Description | Write data to each memory area through the TID mask. |  |  |
| Parameters | Name | Type | Notes |
|  | TIDStr | String | The hexadecimal TID number of the tag |
|  | Mem | byte | memory area to be write, 0- Password area, the first 2 words are the destruction password, the last 2 words are the access password 1- EPC area 2- TID area 3- User area |
|  | WordPtr | byte | Write start word address |
|  | Password | String | Tag's hexadecimal access password, 4 bytes |
|  | wdata | String | The hexadecimal string of the data to be written, the length must be an integer multiple of 4 |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

#### 4.2.7 WriteEPCByTID

| Definition | public int WriteEPCByTID(String TIDStr,String EPCStr,String Password); |  |  |
| --- | --- | --- | --- |
| Description | Rewrite the EPC number of the tag through the TID mask. (Requires the TID start address to start from 0) |  |  |
| Parameters | Name | Type | Notes |
|  | TIDStr | String | The hexadecimal TID number of the tag |
|  | EPCStr | String | The hexadecimal EPC number of the label to be rewritten |
|  | Password | String | Tag's hexadecimal access password, 4 bytes |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

#### 4.2.8 Lock

| Definition | public int Lock(String EPCStr,byte select,byte setprotect,String Password); |  |  |
| --- | --- | --- | --- |
| Description | Set the protection status of each area of the label |  |  |
| Parameters | Name | Type | Notes |
|  | EPCStr | String | The hexadecimal EPC number of the tag |
|  | select | byte | 1 byte, 0x00 - Controls the Kill password read and write protection settings. 0x01 – Control access password read and write protection settings. 0x02 - Controls EPC memory read and write protection settings. 0x03 - Controls TID memory read and write protection settings. 0x04 - Controls user memory read and write protection settings. Other values are reserved. If the reader/writer receives other values, it will return a parameter error message. |
|  | setprotect | byte | 1 byte, When Select is 0x00 or 0x01, the meaning of the SetProtect value is as follows: 0x00 - set to read and write 0x01 - set to always readable and writable 0x02 - set to read and write with password 0x03 - set to never read or write When Select is 0x02, 0x03, 0x04, the meaning of the SetProtect value is as follows: 0x00 - set to writable 0x01 - set to always writable 0x02 - set to writable with password 0x03 - Set to never write. |
|  | Password | String | Tag's hexadecimal access password, 4 bytes |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

#### 4.2.9 Kill

| Definition | public int Kill(String EPCStr,String PasswordStr) |  |  |
| --- | --- | --- | --- |
| Description | This command is used to destroy the label. Once the tag is destroyed, the reader's commands are never processed again. |  |  |
| Parameters | Name | Type | Notes |
|  | EPCStr | String | The hexadecimal EPC number of the tag |
|  | PasswordStr | String | Hex string for label kill password |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

#### 4.2.10 ReadData_G2

| Definition | public int ReadData_G2(byte ENum, byte EPC[], byte Mem,byte WordPtr, byte Num, byte Password[],byte MaskMem,byte MaskAdr[],byte MaskLen,byte[] MaskData, byte Data[],byte Errorcode[]) |  |  |
| --- | --- | --- | --- |
| Description | Read data from each memory area |  |  |
| Parameters | Name | Type | Notes |
|  | ENum | String | 1 byte, the length of the EPC number of the label, when Enum=255, the mask method is used. |
|  | EPC | byte [] | ENum words, EPC number of the label。 |
|  | Mem | byte | memory area to be read, 1- Password area, the first 2 words are the destruction password, the last 2 words are the access password 2- EPC area 3- TID area 3 - User area |
|  | WordPtr | byte | The starting word address for reading |
|  | Num | byte | read word length |
|  | Password | String | Tag's hexadecimal access password, 4 bytes |
|  | MaskMem | byte | 1 byte, mask area. 1 – EPC memory area; 2 – TID storage area; 3 – User memory area. |
|  | MaskAdr | byte [] | 2 bytes, the starting bit address of the mask (unit: Bits). The range is 0~16383. |
|  | MaskLen | byte | 1 byte, the bit length of the mask (unit: Bits). |
|  | MaskData | byte [] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | Data | byte [] | Num words, the read tag data. |
|  | ErrorCode | int [] | When the return value is 0xFC, the label error information is returned. Please check the ErrorCode error code table. |
| Return value (String) | Success: tag data Failed: null; |  |  |
| Sample code |  |  |  |

#### 4.2.11 WriteData_G2

| Definition | public int WriteData_G2(byte WNum, byte ENum,byte EPC[],byte Mem, byte WordPtr, byte Writedata[], byte Password[],byte MaskMem,byte MaskAdr[],byte MaskLen,byte[]MaskData,byte Errorcode[]) |  |  |
| --- | --- | --- | --- |
| Description | Write data to each memory area. |  |  |
| Parameters | Name | Type | Notes |
|  | WNum | byte | 1 byte, the number of words to write, |
|  | ENum | byte | 1 byte, the length of the EPC number of the label. When Enum=255, the mask method is used. |
|  | EPC | byte[] | Enum*2 bytes, the EPC number of the label. |
|  | Mem | byte | memory area to be written, 0- Password area, the first 2 words are the destruction password, the last 2 words are the access password 1- EPC area 2- TID area 3 - User area |
|  | WordPtr | byte | Write start word address |
|  | Writedata | byte[] | Wnum*2 bytes, the data to be written. |
|  | Password | byte[] | 4 bytes, the access password for the tag. |
|  | MaskMem | byte | 1 byte, mask area. 1 – EPC memory area; 2 – TID storage area; 3 – User memory area. |
|  | MaskAdr | byte [] | 2 bytes, the starting bit address of the mask (unit: Bits). The range is 0~16383. |
|  | MaskLen | byte | 1 byte, the bit length of the mask (unit: Bits). |
|  | MaskData | byte [] | n bytes, mask data. n is equal to MaskLen/8. If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded up plus 1. If it is not enough, add 0 in the low order. |
|  | ErrorCode | int [] | When the return value is 0xFC, the label error information is returned. Please check the ErrorCode error code table. |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

#### 4.2.12 Inventory_G2

| Definition | public int Inventory_G2(byte QValue,byte Session,byte AdrTID,byte LenTID,byte Target,byte Ant,byte Scantime,byte pEPCList [],int pOUcharTagNum[],int pListLen[]) |  |  |
| --- | --- | --- | --- |
| Description | Check whether there is an electronic label that conforms to the agreement within the valid range |  |  |
| Parameters | Name | Type | Notes |
|  | QValue | byte | 1 byte, 0~15: The initial Q value used when querying the EPC label, |
|  | Session | byte | 1 byte, the session value used when querying the EPC tag. 0x00: Session uses S0; 0x01: Session uses S1; 0x02: Session uses S2; 0x03: Session uses S3. 0xff: The reader automatically configures the session (only valid for EPC query) It is recommended to select S0 for single or a small number of label queries. |
|  | AdrTID | byte[] | 1 byte, query the starting word address of the TID area. |
|  | LenTID | byte | 1 byte, query the number of data words in the TID area, the range is 1~15, 0 means EPC query |
|  | Target | byte | 1 byte, the Target value of the query tag. 0 – Target A; 1 – Target B. |
|  | Ant | byte[] | 1 byte, the antenna number to be queried this time. 0x80 – Antenna 1; 0x81 – Antenna 2; 0x82 – Antenna 3; 0x83 – Antenna 4; 0x84 – Antenna 5; 0x85 – Antenna 6; 0x86 – Antenna 7; 0x87 – Antenna 8; 0x88 – Antenna 9; 0x89 – Antenna 10; 0x8A – Antenna 11; 0x8B – Antenna 12; 0x8C – Antenna 13; 0x8D – Antenna 14; 0x8E – Antenna 15; 0x8F – Antenna 16; The single-port reader is fixed at 0x80. |
|  | Scantime | byte | 1 byte, the maximum operation time for inventory. The valid range of Scantime is 0 ~ 255, corresponding to (0 ~ 255)*100ms. For Scantime = 0, operation time is not limited. |
|  | pEPCList | byte | The inquired tag data, the data block follows the format stated below: EPC/TID length + EPC/TID No. + RSSI. Data of multiple tags is formed by several identical data blocks in sequence |
|  | pOUcharTagNum | byte [] | The total amount of tag inquired during the current inventory. |
|  | pListLen | byte | The total length of the received data stored in pEPCList. |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

#### 4.2.13 FindEPC

| Definition | public int FindEPC(String EPC,int antenna,List<ReadTag> tagList) |  |  |
| --- | --- | --- | --- |
| Description | Find the epc with antenna |  |  |
| Parameters | Name | Type | Notes |
|  | EPC | String | The hexadecimal EPC number of the tag |
|  | antenna | int | The antenna to find,0x03 means used to antenna 1and 2 |
|  | tagList | List<ReadTag> | Return tag information |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

#### 4.2.14 LedOn_kx2005x

| Definition | public int LedOn_kx2005x(String EPCStr,byte CarrierTime, byte Password[]) |  |  |
| --- | --- | --- | --- |
| Description | Used to controll led tag |  |  |
| Parameters | Name | Type | Notes |
|  | EPC | String | The hexadecimal EPC number of the tag |
|  | CarrierTime | byte | The time of led on. unit 100ms |
|  | Password | byte[] | 4 bytes,Tag’s access password. |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

- 4.3 Custom command

#### 4.3.1 SetCallBack

| Definition | public void SetCallBack(TagCallback callback) |  |  |
| --- | --- | --- | --- |
| Description | Set the callback interface after the inventory is started, and the label data is returned through the callback interface |  |  |
| Parameters | Name | Type | Notes |
|  | callback | TagCallback | Tag’s callback interface |
| Return value (void) |  |  |  |
| Sample code |  |  |  |

#### 4.3.2 SetInventoryParameter

| Definition | public void SetInventoryPatameter(ReaderParameter param) |  |  |
| --- | --- | --- | --- |
| Description | Set the query parameters used when inventory is enabled |  |  |
| Parameters | Name | Type | Notes |
|  | param | ReaderParameter | query parameters |
| Return value (void) |  |  |  |
| Sample code |  |  |  |

#### 4.3.3 GetInventoryParameter

| Definition | public ReaderParameter GetInventoryPatameter() |  |  |
| --- | --- | --- | --- |
| Description | Get the query parameters used in inventory |  |  |
| Parameters | Name | Type | Notes |
| Return value (ReaderParameter) | ReaderParameter |  |  |
| Sample code |  |  |  |

#### 4.3.4 GetUHFInformation

| Definition | public int GetUHFInformation(byte Version[],byte Power[],byte band[],byte MaxFre[],byte MinFre[],byte BeepEn[],int Ant[]) |  |  |
| --- | --- | --- | --- |
| Description | Get basic information of the UHF module. |  |  |
| Parameters | Name | Type | Notes |
|  | Version | byte [] | Output 2 bytes, reader version information. The first byte is the version number, and the second byte is the handle of the subversion number. |
|  | Power | byte [] | Output 1 byte, the output power of the reader. The range is 0 to 30 in dBm. |
|  | band | byte[] | Output 1 byte, spectrum band. 1 - "Chinese band2"; 2 - "US band"; 3 - "Korean band"; 4 - "EU band"; 8 - "Chinese band1"; |
|  | MaxFre | byte[] | Output 1 byte, indicating the current maximum frequency of the reader. |
|  | MinFre | byte[] | Output 1 byte, indicating the current minimum frequency of the reader. |
|  | BeepEn | byte[] | Output 1 byte, buzzer beeps information. |
|  | Ant | int[] | Output 1 word, antenna configuration information. Each bit represents an antenna number, such as 0x0009, the binary is 00000000 00001001, indicating antenna 1 and antenna 4. |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

#### 4.3.5 SetRfPower

| Definition | public int SetRfPower(int Power) |  |  |
| --- | --- | --- | --- |
| Description | Set the reader power |  |  |
| Parameters | Name | Type | Notes |
|  | Power | int | The output power of the reader. The range is 0 to 30 in dBm. The highest bit 7 is 1, which means that the power adjustment is not saved; the bit 0 means that the power is saved and saved. |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

#### 4.3.6 SetAntenna

| Definition | public int SetAntenna(int SetOnce,int AntCfg) |  |  |
| --- | --- | --- | --- |
| Description | Set the effective working antenna of the reader |  |  |
| Parameters | Name | Type | Notes |
|  | SetOnce | int | 1: Indicates that it will not be saved when power off; 0: Indicates power-off save |
|  | AntCfg | byte | Valid antenna number, each bit represents an antenna number, such as 0x0009, binary is 00000000 00001001, indicating antenna 1 and antenna 4. |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

#### 4.3.7 SetGPIO

| Definition | public int SetGPIO(byte GPIO) |  |  |
| --- | --- | --- | --- |
| Description | Set the state of the GPIO port of the reader |  |  |
| Parameters | Name | Type | Notes |
|  | GPIO | byte | GPIO port (Out1-Out2 pin) output status. Bit0-Bit1 control the Out1-Out2 pins respectively, and Bit2-Bit7 are reserved. |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

#### 4.3.8 GetGPIOStatus

| Definition | public int GetGPIOStatus(byte OutputPin[]) |  |  |
| --- | --- | --- | --- |
| Description | Read the GPIO port status of the reader |  |  |
| Parameters | Name | Type | Notes |
|  | OutputPin | byte[] | 1 byte, GPIO port output status. Bit0 represents the pin status of IN1, Bit4-Bit5 represent the status of Out1-Out2 respectively, and other bits are reserved. |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

#### 4.3.9 SetRegion

| Definition | public int SetRegion(int band,int maxfre,int minfre) |  |  |
| --- | --- | --- | --- |
| Description | Set the working frequency band of the reader |  |  |
| Parameters | Name | Type | Notes |
|  | band | Int | 1 byte, spectrum band. 1 - "Chinese band2"; 2 - "US band"; 3 - "Korean band"; 4 - "EU band"; 8 - "Chinese band1"; |
|  | maxfre | Int | Indicates the current maximum frequency of the reader's operation. |
|  | minfre | Int | Indicates the minimum frequency at which the current reader works. |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

The calculation formula of each frequency band:

Chinese band2：		Fs = 920.125 + N * 0.25 (MHz) N∈[0, 19];

US band：			Fs = 902.75 + N * 0.5 (MHz) N∈[0,49];

Korean band：			Fs = 917.1 + N * 0.2 (MHz) N∈[0, 31];

EU band:				Fs = 865.1 + N*0.2(MHz) N∈[0, 14];

Chinese band1：		Fs = 840.125 + N * 0.25 (MHz) N∈[0, 19].

#### 4.3.10 SetBeepNotification

| Definition | public int SetBeepNotification(int BeepEn) |  |  |
| --- | --- | --- | --- |
| Description | This command is used to set the buzzer switch. |  |  |
| Parameters | Name | Type | Notes |
|  | BeepEn | int | 0x00 – disabled beep; 0x01 – enabled beep. |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

#### 4.3.11 SetRfPowerByAnt

| Definition | public int SetRfPowerByAnt(byte Power[]) |  |  |
| --- | --- | --- | --- |
| Description | Set the power according to the antenna port |  |  |
| Parameters | Name | Type | Notes |
|  | Power | byte[] | The power of each antenna port, the number of parameter bytes must be consistent with the number of antenna ports, the low byte represents the power of antenna port 1, and so on |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

#### 4.3.12 GetRfPowerByAnt

| Definition | public int GetRfPowerByAnt(byte Power[]) |  |  |
| --- | --- | --- | --- |
| Description | Get the power according to the antenna port |  |  |
| Parameters | Name | Type | Notes |
|  | Power | byte[] | The power of each antenna port, the number of parameter bytes must be consistent with the number of antenna ports, the low byte represents the power of antenna port 1, and so on |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

#### 4.3.13 SetRelay

| Definition | public int SetRelay(int RelayTime) |  |  |
| --- | --- | --- | --- |
| Description | Used to set the pick-up time of the relay |  |  |
| Parameters | Name | Type | Notes |
|  | RelayTime | int | Relay pick-up time, range 1~255. Unit 50ms, |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

#### 4.3.14 GetDeviceID

| Definition | public String GetDeviceID ()； |  |  |
| --- | --- | --- | --- |
| Description | This command is used to get the unique serial number of the reader。 |  |  |
| Parameters | Name | Type | Notes |
| Return value (String) | Returns a unique serial number hexadecimal string on success, and returns null on failure. |  |  |
| Sample code |  |  |  |

#### 4.3.15 MeasureReturnLoss

| Definition | public int MeasureReturnLoss(byte TestFreq[],byte Ant,byte ReturnLoss[]) |  |  |
| --- | --- | --- | --- |
| Description | This command is used to measure the return loss of the antenna port |  |  |
| Parameters | Name | Type | Notes |
|  | TestFreq | byte[] | 4 bytes, measure the frequency used, in KHz, with the high byte first. The frequency must be a multiple of 125KHz or 100KHz, with the high byte first |
|  | Ant | byte | Measure the antenna port, 0~15 represent antenna 1~antenna 16 respectively |
|  | ReturnLoss | byte[] | 1 byte, the measurement result of the return loss value, in dB. |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

#### 4.3.16 SetWritePower

| Definition | public int SetWritePower(byte WritePower); |  |  |
| --- | --- | --- | --- |
| Description | This command is used to set the power used when writing tags |  |  |
| Parameters | Name | Type | Notes |
|  | WritePower | byte | Write power parameter, the highest bit 7 enable flag, 0-not enabled; 1-enabled. Bit0~bit6 represent the write power value, the range is 0~33, and the unit is dbm. |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

#### 4.3.17 GetWritePower

| Definition | public int GetWritePower(byte[] WritePower); |  |  |
| --- | --- | --- | --- |
| Description | This command is used to read and write the power used by the tag |  |  |
| Parameters | Name | Type | Notes |
|  | WritePower | byte[] | 1 byte, write power parameter, the highest bit 7 enable flag, 0-not enabled; 1-enabled. Bit0~bit6 represent the write power value, the range is 0~33, and the unit is dbm. |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

#### 4.3.18 SetCheckAnt

| Definition | public int SetCheckAnt(byte CheckAnt); |  |  |
| --- | --- | --- | --- |
| Description | This command is used to set whether to enable the antenna detection function |  |  |
| Parameters | Name | Type | Notes |
|  | CheckAnt | byte | Antenna detection switch, 0-off; 1-on |
| Return value (int) | Success:0; |  |  |
| Sample code |  |  |  |

- 4.4 Return value error code table

| Error code | Description |
| --- | --- |
| 0x00 | API is called successfully. |
| 0x01 | No find tag |
| 0x05 | Access password error. |
| 0x09 | Kill password error. |
| 0x0A | All-zero tag killing password is invalid. |
| 0x0B | Command is not support by the tag |
| 0x0C | All-zero tag access password is invalid for such command. |
| 0x0D | Fail to setup read protection for a protection enabled tag. |
| 0x0E | Fail to unlock a protection disabled tag. |
| 0x10 | Some bytes stored in the tag are locked. |
| 0x11 | Lock operation failed. |
| 0x12 | Already locked, lock operation failed. |
| 0x13 | Fail to store the value of some preserved parameters. Configuration will still valid before reader shut down. |
| 0x14 | Modification failed. |
| 0xF8 | Error detected in antenna check. |
| 0xF9 | Operation failed. |
| 0xFA | Tag is detected, but fails to complete operation due to poor communication. |
| 0xFB | No tag is detected. |
| 0xFC | Error code returned from tags. |
| 0xFD | Command length error. |
| 0xFE | Illegal command. |
| 0xFF | Parameter error. |
| 0x30 | Communication error. |