UHFReader288.DLL

Dynamic link library manual

V3.0

# Contents

Contents	2

1	Introduction	5

2	SDK summary	6

3	Programming interface	7

3.1	Connect/disconnect reader	7

3.1.1	AutoOpenComPort	7

3.1.2	OpenComPort	8

3.1.3	CloseComPort	9

3.1.4	CloseSpecComPort	10

3.1.5	OpenNetPort	11

3.1.6	CloseNetPort	12

3.2	18000-6C commands	13

3.2.1	Inventory_G2	13

3.2.2	InventoryMix_G2	16

3.2.3	InitRFIDCallBack	19

3.2.4	ReadData_G2	20

3.2.5	WriteData_G2	22

3.2.6	BlockWrite_G2	24

3.2.7	ExtReadData_G2	26

3.2.8	ExtWriteData_G2	28

3.2.9	WriteEPC_G2	30

3.2.10	BlockErase_G2	31

3.2.11	Lock_G2	32

3.2.12	KillTag _G2	34

3.2.13	SingleTagInventory_G2	35

3.2.14. StartRead	36

3.2.15. StopRead	37

3.2.16. GetRfidTagData	38

3.3	Buffer operation commands	39

3.3.1	InventoryBuffer_G2	39

3.3.2	ReadBuffer_G2	41

3.3.3	ClearBuffer_G2	42

3.3.4	GetBufferCnt_G2	43

3.3.5	 SetSaveLen	44

3.3.5	GetSaveLen	45

3.4	NXP Tag commands	46

3.4.1	SetPrivacyByEPC _G2	46

3.4.2	SetPrivacyWithoutEPC_G2	47

3.4.3	ResetPrivacy_G2	48

3.4.4	CheckPrivacy_G2	49

3.4.5	EASConfigure_G2	50

3.4.6	EASAlarm_G2	51

3.5	Impinj Monza4 QT tag commands	52

3.5.1	GetMonza4QTWorkParamter_G2	52

3.5.2	SetMonza4QTWorkParamter_G2	53

3.5.3	Inventory_QT_G2	54

3.5.4	SetTagCustomFunction_G2	56

3.6	18000-6B Commands	57

3.6.1	InventorySingle_6B	57

3.6.2	InventoryMultiple_6B	58

3.6.3	ReadData_6B	60

3.6.4	WriteData_6B	61

3.6.5	CheckLock_6B	62

3.6.6	Lock_6B	63

3.7	Reader customised commands	64

3.7.1	GetReaderInformation	64

3.7.2	SetAddress	66

3.7.3	SetInventoryScanTime	67

3.7.4	SetRfPower	68

3.7.5	SetRegion	69

3.7.6	SetBaudRate	71

3.7.7	BuzzerAndLEDControl	72

3.7.8	SetAntennaMultiplexing	73

3.7.9	SetAntenna	74

3.7.10	SetBeepNotification	76

3.7.11	SetReal_timeClock	77

3.7.12	GetTime	78

3.7.13	SetRelay	79

3.7.14	GetGPIOStatus	80

3.7.15	SetGPIO	81

3.7.16	SetNotificationPulseOutput	82

3.7.17	SetTIDParameter	83

3.7.18	SetQS	84

3.7.19	GetQS	85

3.7.20	GetSeriaNo	86

3.7.21	SetCheckAnt	87

3.7.22	SetWorkMode	88

3.7.23	GetSystemParameter	89

3.7.24	ReadActiveModeData	90

3.7.25	SetEASSensitivity	91

3.7.26	SetMask	92

3.7.27	SetResponsePamametersofAuto_runningMode	93

3.7.28	SetInventoryInterval	94

3.7.29	SelectTagType	95

3.7.30	GetTagBufferInfo	96

3.7.31	ClearTagBuffer	97

3.7.32	GetModuleVersion	98

3.7.33	SetFlashRom	99

3.7.34	WriteRfPower	100

3.7.35	ReadRfPower	101

3.7.36	RetryTimes	102

3.7.37	SetReadMode	103

3.7.38	SetReadParameter	104

3.7.39	GetReadParameter	106

3.7.40	SetRfPowerByAnt	106

3.7.41	GetRfPowerByAnt	108

3.7.42	SetCfgParameter	109

3.7.43	GetCfgParameter	110

3.7.44	SetProfile	111

3.7.45	StopImmediately	113

Appendix 1: Reader Error code table	114

Appendix 2: Tag ErrorCode table	115

# 1	Introduction

The SDK is designed for C, C++ and other C function interface compatible languages, such as Delphi, C#, VB6.0 and VB.NET etc. It supports both 32-bit and 64-bit WINDOWS operation system.

By loading the "UHFReader288.DLL", host application can control the 18000-6C and 18000-6B formats readers. Host application can access multiple readers via different communication ports simultaneously.

# 2	SDK summary

This SDK includes the following contents:

| Files | Description |
| --- | --- |
| UHFReader288.dll | Dynamic Link Library (DLL), including all the API interfaces. |
| UHFReader288.lib | Static library for VC++ project. |
| UHFReader288.h | Function declaration, C/C++ project header files. |
| Reader.h | C/C++ project additional header files. |
| RWDev.cs | C# function class files. |

| Programming language | Description |
| --- | --- |
| VC++ | Copy " UHFReader288.lib", " UHFReader288.h", "Reader.h" to project directory and add these files to project. Copy " UHFReader288.dll" to the exe file output directory. |
| C# | Copy "RWDev.cs" to project directory and add the file to project. Copy " UHFReader288.dll" to the exe file output directory. |

# 3	Programming interface

## 3.1	Connect/disconnect reader

3.1.1	AutoOpenComPort

| Definition | int AutoOpenComPort(int* port, BYTE* ComAdr, BYTE baud, int* FrmHandle); |  |  |
| --- | --- | --- | --- |
| Description | Automatically identify reader connected serial port and initialise serial connection to reader. This function only supports serial from COM1 to COM 9. |  |  |
| Parameters | Name | Type | Notes |
|  | port | Int* | Serial port number for reader connection. 1 - COM1; 2 - COM2, etc. |
|  | ComAdr | BYTE* | Target reader address. In the case when reader address is unknown, the broadcasting address 0xFF can be used to initialise the serial connection. Once the function is called successfully, the actual address of the reader will be returned as the value of this parameter. |
|  | baud | BYTE | Serial baud rate. 0 - 9600 bps; 1 - 19200 bps; 2 - 38400 bps; 5 - 57600 bps; 6 - 115200 bps. |
|  | FrmHandle | Int* | Reader connected handle will be returned as the value of this parameters. The value of handle will be used in all the future API operation. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |
| Sample code | int port=0; BYTE address=255;// broadcasting address BYTE baud=0;//19200 int FrmHandle=-1; int fCmdRet =AutoOpenComPort(&port,&address, baud, &FrmHandle); |  |  |

3.1.2	OpenComPort

| Definition | int OpenComPort(int port, BYTE* ComAdr, BYTE baud, int* FrmHandle) ; |  |  |
| --- | --- | --- | --- |
| Description | Open the serial connection to reader |  |  |
| Parameters | Name | Type | Notes |
|  | port | Int | Serial port number for reader connection. 1 - COM1; 2 - COM2, etc. |
|  | ComAdr | BYTE* | Target reader address. In the case when reader address is unknown, the broadcasting address 0xFF can be used to initialise the serial connection. Once the function is called successfully, the actual address of the reader will be returned as the value of this parameter. |
|  | baud | BYTE | Serial baud rate. 0 - 9600 bps; 1 - 19200 bps; 2 - 38400 bps; 5 - 57600 bps; 6 - 115200 bps. |
|  | FrmHandle | Int* | Reader connected handle will be returned as the value of this parameters. The value of handle will be used in all the future API operation. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |
| Sample code | int port=1;//open serial com1 BYTE address=255;//broadcasting address BYTE baud=5;//57600 int FrmHandle=-1; int fCmdRet =OpenComPort(port,&address, baud, &FrmHandle); if(fCmdRet==0) { // Reader is connected successfully ........... } |  |  |

3.1.3	CloseComPort

| Definition | int CloseComPort(); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Close all ports to disconnect with reader and release associated resource. |  |  |  |
| Parameters (N/A) | Name | Type | Orientation | Notes |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to error code table) . |  |  |  |
| Sample code | N/A |  |  |  |

3.1.4	CloseSpecComPort

| Definition | int CloseSpecComPort(int FrmHandle); |  |  |
| --- | --- | --- | --- |
| Description | Close a specific port to disconnect with reader and release associated resource. |  |  |
| Parameters | Name | Type | Notes |
|  | FrmHandle | int | Reader connected handle, obtained from the OpenComPort function. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to error code table) . |  |  |
| Sample code | N/A |  |  |

3.1.5	OpenNetPort

| Definition | Int OpenNetPort(int Port,LPCTSTR IPaddr, BYTE*ComAdr, int *Frmhandle); |  |  |
| --- | --- | --- | --- |
| Description | Open the TCP connection to reader |  |  |
| Parameters | Name | Type | Notes |
|  | port | int | TCP port number for reader connection. |
|  | IPaddr | LPCTSTR | IP address of the reader. |
|  | ComAdr | BYTE* | Target reader address, or simply use broadcasting address 0xff for TCP connection. |
|  | FrmHandle | Int* | Reader connected handle will be returned as the value of this parameters. The value of handle will be used in all the future API operation. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |
| Sample code | int port=6000; LPCTSTR ipAddr =”192.168.1.190”; BYTE ComAdr =255; // broadcasting address int FrmHandle=-1; int fCmdRet =OpenNetPort(port, ipAddr ,&ComAdr, &FrmHandle); if(fCmdRet==0) { // Reader is connected successfully ...... } |  |  |

3.1.6	CloseNetPort

| Definition | int CloseNetPort(int FrmHandle); |  |  |
| --- | --- | --- | --- |
| Description | Close the TCP connection with reader and release associated resource |  |  |
| Parameters | Name | Type | Notes |
|  | FrmHandle | int | Reader connected handle, obtained from the OpenNetPort function. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |
| Sample code | N/A |  |  |

## 3.2	18000-6C commands

3.2.1	Inventory_G2

| Definition | int Inventory_G2(BYTE * ComAddr,BYTE QValue,BYTE Session,BYTE MaskMem,BYTE *MaskAdr,BYTE MaskLen,BYTE *MaskData,BYTE MaskFlag,BYTE AdrTID,BYTE LenTID,BYTE TIDFlag,BYTE Target,BYTE InAnt,BYTE Scantime,BYTE FastFlag,BYTE *pEPCList,BYTE *Ant,int *Totallen, int *CardNum,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Inquire RFID tags with corresponding protocol within the effective field. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | QValue | BYTE | [in] | 1 byte. bit7: Statistic data packet flag; 0 – after inventory，DO NOT deliver statistic data packet of inventory process; 1 – after inventory，deliver statistic data packet of inventory process. bit5: FastID inventory indicator. This function is used to rapidly obtain tag EPC and TID, only supported by some tags from the Impinj Monza series; 0 – disable;1 – enable. Bit4: phase flag 0 – disable;1 – enable. Bit3 ~ bit0: the original Q-value of EPC tag inventory. NOTE: 1. The setting of Q-value should follow the rule: 2Q ≈ total amount of tags within the effective field. The range of Q-value is 0 ~ 15, if other value is delivered in this field, reader will return a parameter error status in the response frame. 2. Inventory with statistic data packet is SLOWER than inventory without statistic data packet. |
|  | Session | BYTE | [in] | 1 byte, the Session-value of EPC tag inventory. 0x00 – apply S0 as Session value; 0x01 – apply S1 as Session value; 0x02 – apply S2 as Session value; 0x03 – apply S3 as Session value. 0xff – apply reader smart configuration (only valid in EPC inventory). Inventory for single tag or small amount of tag, S0 is the recommended setting. |
|  | MaskMem | BYTE | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | BYTE | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | BYTE* | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | MaskFlag | BYTE | [in] | 1 byte, mask flag. 0 – disable mask; 1 – enable mask. |
|  | AdrTID | BYTE | [in] | 1 byte, entry address of inventory TID memory. |
|  | LenTID | BYTE | [in] | 1 byte, data length for TID inventory operation, the valid range of LenTID is 0 ~ 15. |
|  | TIDFlag | BYTE | [in] | 1 byte, inventory purpose indicator. 0 – EPC inventory; 1 – TID inventory. |
|  | Target | BYTE | [in] | 1 byte, the Target value of EPC tag inventory. 0x00 – apply Target A; 0x01 – apply Target B. |
|  | InAnt | BYTE | [in] | 1 byte, antenna selection for the current inventory. 0x80 – antenna 1; 0x81 – antenna 2; 0x82 – antenna 3; 0x83 – antenna 4; 0x84 – antenna 5; 0x85 – antenna 6; 0x86 – antenna 7; 0x87 – antenna 8; 0x88 – antenna 9; 0x89 – antenna 10; 0x8A – antenna 11; 0x8B – antenna 12. InAnt is 0x08 for single port reader. |
|  | Scantime | BYTE | [in] | 1 byte, the maximum operation time for inventory. The valid range of Scantime is 0 ~ 255, corresponding to (0 ~ 255)*100ms. For Scantime = 0, operation time is not limited. |
|  | FastFlag | BYTE | [in] | 1 byte, express inventory indicator 0 – disable express inventory, Target, InAnt and Scantime are NOT essential, can be set to default value 0; 1 – enable express inventory, Target, InAnt and Scantime are need to be defined. |
|  | pEPCList | BYTE* | [out] | The inquired tag data, the data block follows the format stated below: EPC/TID length + EPC/TID No. + RSSI. Data of multiple tags is formed by several identical data blocks in sequence. If length bit6=1.add 4 bytes phase information +3 bytes freqkhz after rssi |
|  | Ant | BYTE* | [out] | 1 byte indicates which antenna had inquired a tag. 4 antenna ports reader: Every bit in Ant represents one corresponding antenna. For example, 0x04 is 0000 0100 in binary. This indicates Antenna 3 had inquired this specific tag. 12 antenna ports reader Ant value 0 ~ 11 corresponding to Antenna 1 to Antenna 12. For example, Ant = 0 represents Antenna 1 had inquired this specific tag. |
|  | Totallen | int* | [out] | The total length of the received data stored in pEPCList. |
|  | CardNum | int* | [out] | The total amount of tag inquired during the current inventory. Note: Totallen and CardNum are 0 for call back return. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | 0x01: inventory completed, tag data will be delivered; 0x02: inventory timeout, operation is aborted, inquired tag data will be delivered. 0xF8: error occurred in antenna check, the target antenna might be disconnected. For description on other error, refer to reader error code table. |  |  |  |
| Sample code | N/A |  |  |  |

3.2.2	InventoryMix_G2

| Definition | int InventoryMix_G2(BYTE * ComAddr,BYTE QValue,BYTE Session,BYTE MaskMem,BYTE *MaskAdr,BYTE MaskLen,BYTE *MaskData,BYTE MaskFlag,BYTE ReadMem,BYTE* ReadAdr,BYTE ReadLen,BYTE* Psd,BYTE Target,BYTE InAnt,BYTE Scantime,BYTE FastFlag,BYTE *pEPCList,BYTE *Ant,int *Totallen, int *CardNum,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Inquire RFID tags with corresponding protocol within the effective field, and obtain specific tag data once EPC number is inquired. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | QValue | BYTE | [in] | 1 byte. bit7: Statistic data packet flag. 0 – after inventory，DO NOT deliver statistic data packet of inventory process; 1 – after inventory，deliver statistic data packet of inventory process. Bit4: phase flag 0 – disable;1 – enable. Bit3 ~ bit0: the original Q-value of EPC tag inventory. NOTE: 1. The setting of Q-value should follow the rule: 2Q ≈ total amount of tags within the effective field. The range of Q-value is 0 ~ 15, if other value is delivered in this field, reader will return a parameter error status in the response frame. 2. Inventory with statistic data packet is SLOWER than inventory without statistic data packet. |
|  | Session | BYTE | [in] | 1 byte, the Session-value of inventory. 0x00 – apply S0 as Session value; 0x01 – apply S1 as Session value; 0x02 – apply S2 as Session value; 0x03 – apply S3 as Session value. Inventory for single tag or small amount of tag, S0 is the recommended setting. |
|  | MaskMem | BYTE | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | BYTE | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | BYTE* | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | MaskFlag | BYTE | [in] | 1 byte, mask flag. 0 – disable mask; 1 – enable mask. |
|  | ReadMem | BYTE | [in] | 1 byte, the type of target memory. 0x00 – reserved memory; 0x01 – EPC memory; 0x02 – TID memory; 0x03 – user memory. All other values are reserved, reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | ReadAdr | BYTE* | [in] | 2 bytes, the initial word address of target, most- significant byte first. |
|  | ReadLen | BYTE | [in] | 1 byte, the amount of word to be read in this operation. |
|  | Psd | BYTE* | [in] | 4 bytes, the access password. |
|  | Target | BYTE | [in] | 1 byte, the Target value of tag inventory. 0x00 – apply Target A; 0x01 – apply Target B. |
|  | InAnt | BYTE | [in] | 1 byte, antenna selection for the current inventory. 0x80 – antenna 1; 0x81 – antenna 2; 0x82 – antenna 3; 0x83 – antenna 4; 0x84 – antenna 5; 0x85 – antenna 6; 0x86 – antenna 7; 0x87 – antenna 8; 0x88 – antenna 9; 0x89 – antenna 10; 0x8A – antenna 11; 0x8B – antenna 12. InAnt is 0x08 for single port reader. |
|  | Scantime | BYTE | [in] | 1 byte, the maximum operation time for inventory. The valid range of Scantime is 0 ~ 255, corresponding to (0 ~ 255)*100ms. For Scantime = 0, operation time is not limited. |
|  | FastFlag | BYTE | [in] | 1 byte, express inventory indicator 0 – disable express inventory, Target, InAnt and Scantime are NOT essential, can be set to default value 0. 1 – enable express inventory, Target, InAnt and Scantime are need to be defined. |
|  | pEPCList | BYTE* | [out] | The inquired tag data, the data block follows the format stated below: Rolling code + EPC length/data length + EPC No. /data + RSSI. Data of multiple tags is formed by several identical data blocks in sequence. The most-significant bit of rolling code: 0 – this block contains EPC; 1 – this block contains tag memory data. Firstly EPC will be uploaded, followed by tag memory data. If two sets of lower 7 bits of rolling code are consecutive, the corresponding data comes from the same tag. The lower 7 bits of rolling code ranges from 0 to 127. The code will increase for every block uploading and will recount again from 0 after it exceeds 127. If length bit6=1.add 4 bytes phase information +3 bytes freqkhz after rssi |
|  | Ant | BYTE* | [out] | 1 byte indicates which antenna had inquired a tag. 4 antenna ports reader: Every bit in Ant represents one corresponding antenna. For example, 0x04 is 0000 0100 in binary. This indicates Antenna 3 had inquired this specific tag. 12 antenna ports reader Ant value 0 ~ 11 corresponding to Antenna 1 to Antenna 12. For example, Ant = 0 represents Antenna 1 had inquired this specific tag. |
|  | Totallen | int* | [out] | The total length of the received data stored in pEPCList. |
|  | CardNum | int* | [out] | The total amount of tag inquired during the current inventory. Note: Totallen and CardNum are 0 for call back return. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | 0x01: inventory completed, tag data will be delivered; 0x02: inventory timeout, operation is aborted, inquired tag data will be delivered. 0xF8: error occurred in antenna check, the target antenna might be disconnected. For description on other error, refer to reader error code table. |  |  |  |
| Sample code | N/A |  |  |  |

3.2.3	InitRFIDCallBack

| Definition | void InitRFIDCallBack(pRFIDCallBack pUIDback,BOOL isBackUID,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Initialise call back function, after the initialisation of this API, tag inquired from Inventory and Mix Inventory commands can be uploaded via call back. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | pUIDback | pRFIDCallBack | [in] | Call back interface pointer. typedef struct _RFIDTag { // packet type indicator BYTE PacketParam; BYTE LEN; // length LPCTSTR UID; //EPC or UID BYTE RSSI; BYTE ANT; int FrmHandle; }MyStruct,*RFIDTagStruct; Typedefvoid(CALLBACK*pRFIDCallBack)( RFIDTagStruct lpRfid, INT32 nEvt); |
|  | isBackUID | BOOL | [in] | Reserved. |
|  | FrmHandle | DWORD | [in] | Reader connected handle, obtained from open port commands. |
| Return value | N/A |  |  |  |
| Sample code | Refer to Examples\C#\multiple antenna test\ MutipleAntDemo |  |  |  |

3.2.4	ReadData_G2

| Definition | int ReadData_G2(BYTE * ComAddr, BYTE *EPC, BYTE ENum, BYTE Mem, BYTE WordPtr, BYTE Num, BYTE *Password, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, BYTE *Data, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Read a part of data or the whole memory area from tag reserved memory, EPC memory, TID memory, user memory. Read operation starts from a specific address. The unit of data is word. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | BYTE* | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | BYTE | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Mem | BYTE | [in] | 1 byte, the type of target memory. 0x00 – reserved memory 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. All other values are reserved. Reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | WordPtr | BYTE | [in] | 1 byte, the initial word address of target. |
|  | Num | BYTE | [in] | 1 byte, the amount of words to be read in this operation. |
|  | Password | BYTE* | [in] | 4 bytes, the access password. |
|  | MaskMem | BYTE | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | BYTE | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | BYTE* | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | Data | BYTE* | [out] | Inquired tag data, with a length stated in Num. |
|  | ErrorCode | int * | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.5	WriteData_G2

| Definition | int WriteData_G2(BYTE *ComAdr, BYTE * EPC, BYTE Wnum, BYTE Enum, BYTE Mem, BYTE WordPtr,BYTE*Writedata,BYTE* Password, BYTE MaskMem,BYTE*MaskAdr,BYTE MaskLen,BYTE * MaskData,int * errorcode,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Write multiple words to reserved memory, EPC memory, TID memory or user memory in one communication cycle. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | BYTE* | [in] | Tag EPC number, with a length stated in Enum. |
|  | Wnum | BYTE | [in] | 1 byte, the amount of words to be written. For ENum = 255, mask will be enabled. |
|  | Enum | BYTE | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Mem | BYTE | [in] | 1 byte, the type of target memory. 0x00 – reserved memory 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. All other values are reserved. Reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | WordPtr | BYTE | [in] | 1 byte, the initial word address of target. |
|  | Writedata | BYTE* | [in] | Data to be written to tag, with a length stated in WNum. |
|  | Password | BYTE* | [in] | 4 bytes, the access password. |
|  | MaskMem | BYTE | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | BYTE | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | BYTE* | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | int * | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.6	BlockWrite_G2

| Definition | int BlockWrite_G2(BYTE *ComAdr, BYTE * EPC, BYTE Wnum, BYTE Enum, BYTE Mem, BYTE WordPtr,BYTE*Writedata,BYTE* Password, BYTE MaskMem,BYTE*MaskAdr,BYTE MaskLen,BYTE * MaskData,int * errorcode,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Write multiple words to reserved memory, EPC memory, TID memory or user memory in one communication cycle. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | BYTE* | [in] | Tag EPC number, with a length stated in Enum. |
|  | Wnum | BYTE | [in] | 1 byte, the amount of words to be written. For ENum = 255, mask will be enabled. |
|  | Enum | BYTE | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Mem | BYTE | [in] | 1 byte, the type of target memory. 0x00 – reserved memory 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. All other values are reserved. Reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | WordPtr | BYTE | [in] | 1 byte, the initial word address of target. |
|  | Writedata | BYTE* | [in] | Data to be written to tag, with a length stated in WNum. |
|  | Password | BYTE* | [in] | 4 bytes, the access password. |
|  | MaskMem | BYTE | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | BYTE | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | BYTE* | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | int * | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.7	ExtReadData_G2

| Definition | int ExtReadData_G2(BYTE * ComAddr, BYTE *EPC, BYTE ENum, BYTE Mem, BYTE* WordPtr, BYTE Num, BYTE *Password, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, BYTE *Data, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Read data from reserved memory, EPC memory, TID memory, user memory of tag. Read operation starts from a defined address, and the unit of data is word. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | BYTE* | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | BYTE | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Mem | BYTE | [in] | 1 byte, the type of target memory. 0x00 – reserved memory 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. All other values are reserved. Reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | WordPtr | BYTE* | [in] | 2 bytes, the initial word address of target, most- significant byte first. |
|  | Num | BYTE | [in] | 1 byte, the amount of word to be read in this operation. |
|  | Password | BYTE* | [in] | 4 bytes, the access password. |
|  | MaskMem | BYTE | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | BYTE | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | BYTE* | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | Data | BYTE* | [out] | Inquired tag data, with a length stated in Num. |
|  | ErrorCode | int * | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.8	ExtWriteData_G2

| Definition | int WriteData_G2(BYTE *ComAdr, BYTE * EPC, BYTE Wnum, BYTE Enum, BYTE Mem, BYTE* WordPtr,BYTE*Writedata,BYTE* Password, BYTE MaskMem,BYTE*MaskAdr, BYTE MaskLen,BYTE * MaskData,int * errorcode,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Write multiple words to reserved memory, EPC memory, TID memory or user memory in one communication cycle. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | BYTE* | [in] | Tag EPC number, with a length stated in Enum. |
|  | Wnum | BYTE | [in] | 1 byte, the amount of words to be written. |
|  | Enum | BYTE | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Mem | BYTE | [in] | 1 byte, the type of target memory. 0x00 – reserved memory 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. All other values are reserved. Reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | WordPtr | BYTE* | [in] | 2 byte, the initial word address of target. |
|  | Writedata | BYTE* | [in] | Data to be written to tag, with a length stated in WNum. |
|  | Password | BYTE* | [in] | 4 bytes, the access password. |
|  | MaskMem | BYTE | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | BYTE | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | BYTE* | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | int * | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.9	WriteEPC_G2

| Definition | int WriteEPC_G2(BYTE * ComAddr,BYTE *Password, BYTE *EPC, BYTE ENum, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Write EPC number to a tag. During the operation, only one tag is allowed to be place in the antenna effective area. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | Password | BYTE* | [in] | 4 bytes, the access password. |
|  | EPC | BYTE* | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | BYTE | [in] | 1 byte, the length of EPC number. |
|  | ErrorCode | int * | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.10	BlockErase_G2

| Definition | int BlockErase_G2(BYTE * ComAddr, BYTE *EPC, BYTE ENum, BYTE Mem, BYTE WordPtr, BYTE Num, BYTE *Password, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Erase multiple words from reserved memory, EPC memory, TID memory or user memory. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | BYTE* | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | BYTE | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Mem | BYTE | [in] | 1 byte, the type of target memory. 0x00 – reserved memory 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. All other values are reserved. Reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | WordPtr | BYTE* | [in] | 1 byte, the initial word address of target. |
|  | Num | BYTE | [in] | 1 byte, the amount of words to be erased in this operation. |
|  | Password | BYTE* | [in] | 4 bytes, the access password. |
|  | MaskMem | BYTE | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | BYTE | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | BYTE* | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | int * | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.11	Lock_G2

| Definition | int Lock_G2(BYTE * ComAddr,BYTE *EPC, BYTE ENum, BYTE select, BYTE setprotect, BYTE *Password,BYTE MaskMem,BYTE *MaskAdr,BYTE MaskLen,BYTE *MaskData, int *Errorcode,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | The read/write protection status for the following memories: • Reserved memory Readable/ writable without protection, permanently readable/ writable, readable/writeable with password protected or permanently unreadable/ unwritable; • EPC memory /user reserved memory Writable without protection, permanently writable, writeable with password protected or permanently unwritable; EPC memory, user reserved memory and TID memory are permanently readable. Furthermore TID memory is readable only, permanently unwritable. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | BYTE* | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | BYTE | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | select | BYTE | [in] | 1 byte. 0x00 – modify kill password read/write protection; 0x01 – modify access password read/write protection; 0x02 – modify EPC memory read/write protection; 0x03 – modify TID memory read/write protection; 0x04 – modify user memory read/write protection. All other values are reserved, reader will not execute the command and will return a parameter error status in the response frame if other value is delivered in this field. |
|  | setprotect | BYTE | [in] | 1 byte. For Select = 0x00 or 0x01: the kill password or access password protection setting. SetProtect = 0x00 – set to readable/ writable without protection; 0x01 – set to permanently readable/ writable; 0x02 – set to readable/writeable with password protected; 0x03 – set to permanently unreadable/ unwritable. For Select = 0x02, 0x03, and 0x04: the EPC, TID and user memory protection setting. SetProtect = 0x00 – set to writable without protection; 0x01 – set to permanently writable; 0x02 – set to writeable with password protected; 0x03 – set to permanently unwritable. All other values of Select and SetProtect are reserved, reader will not execute the command and will return a parameter error status in the response frame if other value is delivered in this field. |
|  | Password | BYTE* | [in] | 4 bytes, the access password. |
|  | MaskMem | BYTE | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | BYTE | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | BYTE* | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | int * | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.12	KillTag _G2

| Definition | int KillTag_G2(BYTE * ComAddr, BYTE *EPC, BYTE ENum, BYTE *Killpwd, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Tag killing command. After the kill operation, the tags will no longer process any command from reader. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | BYTE* | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | BYTE | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Killpwd | BYTE* | [in] | 4 bytes, the kill password. |
|  | MaskMem | BYTE | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | BYTE | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | BYTE* | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | int * | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.13	SingleTagInventory_G2

| Definition | int SingleTagInventory_G2(BYTE * ComAddr, BYTE* EPC, int *EPCLength, int *CardNum,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Inquire RFID tag with corresponding protocol within the effective field. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | BYTE* | [out] | EPCLength+2 bytes, follows the format stated below: Antenna number + EPC No. + RSSI. |
|  | EPCLength | int* | [out] | 1 byte, the length of EPC number. |
|  | CardNum | int* | [out] | 1 byte, amount of tag inquired during the current inventory. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.14. StartRead

| Definition | int StartRead(BYTE * ComAddr, BYTE Target, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Start fast inventory,the data get by GetRfidTagData. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | Target | BYTE | [in] | Inventory flag，0-Target A，1-Target B |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.15. StopRead

| Definition | int StopRead(BYTE * ComAddr, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Stop fast inventory. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.2.16. GetRfidTagData

| Definition | int GetRfidTagData(BYTE * ComAddr, BYTE *data，int*length,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Get data of fast inventory mode, Analysis according to the user manual protocol. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | data | BYTE* | [out] | tag data |
|  | length | int* | [out] | The length of data, |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

## 3.3	Buffer operation commands

3.3.1	InventoryBuffer_G2

| Definition | int InventoryBuffer_G2(BYTE * ComAddr,BYTE QValue,BYTE Session,BYTE MaskMem,BYTE *MaskAdr,BYTE MaskLen,BYTE *MaskData,BYTE MaskFlag,BYTE AdrTID,BYTE LenTID,BYTE TIDFlag,BYTE Target,BYTE InAnt,BYTE Scantime,BYTE FastFlag ,int * BufferCount,int *CardNum,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Inquire RFID tag with corresponding protocol within the effective field. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | QValue | BYTE | [in] | 1 byte. The original Q-value of EPC tag inventory. NOTE: 1. The setting of Q-value should follow the rule: 2Q ≈ total amount of tags within the effective field. The range of Q-value is 0 ~ 15, if other value is delivered in this field, reader will return a parameter error status in the response frame. |
|  | Session | BYTE | [in] | 1 byte, the Session-value of EPC tag inventory. 0x00 – apply S0 as Session value; 0x01 – apply S1 as Session value; 0x02 – apply S2 as Session value; 0x03 – apply S3 as Session value. 0xff – apply reader smart configuration (only valid in EPC inventory). Inventory for single tag or small amount of tag, S0 is the recommended setting. |
|  | MaskMem | BYTE | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | BYTE | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | BYTE* | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | MaskFlag | BYTE | [in] | 1 byte, mask flag. 0 – disable mask; 1 – enable mask. |
|  | AdrTID | BYTE | [in] | 1 byte, entry address of inventory TID memory. |
|  | LenTID | BYTE | [in] | 1 byte, data length for TID inventory operation, the valid range of LenTID is 0 ~ 15. |
|  | TIDFlag | BYTE | [in] | 1 byte, inventory purpose indicator. 0 – EPC inventory; 1 – TID inventory. |
|  | Target | BYTE | [in] | 1 byte, the Target value of EPC tag inventory. 0x00 – apply Target A; 0x01 – apply Target B. |
|  | InAnt | BYTE | [in] | 1 byte, antenna selection for the current inventory. 0x80 – antenna 1; 0x81 – antenna 2; 0x82 – antenna 3; 0x83 – antenna 4; 0x84 – antenna 5; 0x85 – antenna 6; 0x86 – antenna 7; 0x87 – antenna 8; 0x88 – antenna 9; 0x89 – antenna 10; 0x8A – antenna 11; 0x8B – antenna 12. InAnt is 0x08 for single port reader. |
|  | Scantime | BYTE | [in] | 1 byte, the maximum operation time for inventory. The valid range of Scantime is 0 ~ 255, corresponding to (0 ~ 255)*100ms. For Scantime = 0, operation time is not limited. |
|  | FastFlag | BYTE | [in] | 1 byte, express inventory indicator 0 – disable express inventory, Target, InAnt and Scantime are NOT essential, can be set to default value 0; 1 – enable express inventory, Target, InAnt and Scantime are need to be defined. |
|  | BufferCount | int* | [out] | 2 bytes, the total amount of tag stored in the memory buffer, tags with identical EPC/TID data will be treated as one tag. BufferCount is the sum of tag amount from multiple inventories, until the memory buffer is being cleared. |
|  | CardNum | int* | [out] | 2 bytes, the amount of tag inquired in the current inventory, a tag being accessed multiple times will also increase the amount. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | 0x01: inventory completed, tag data will be delivered; 0x02: inventory timeout, operation is aborted, inquired tag data will be delivered. 0xF8: error occurred in antenna check, the target antenna might be disconnected. For description on other error, refer to reader error code table. |  |  |  |
| Sample code | N/A |  |  |  |

3.3.2	ReadBuffer_G2

| Definition | int ReadBuffer_G2(BYTE *ComAdr, int *Totallen, int *CardNum, BYTE *pEPCList, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain all the tag information from reader memory. This command will not cause affection on the status of the stored data. Data can be access multiple times. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | Totallen | int* | [out] | The total length of the received data stored in pEPCList. |
|  | CardNum | int * | [out] | The total amount of tag inquired during the current inventory. |
|  | pEPCList | BYTE* | [out] | The inquired tag data, with a length stated in Totallen. Data of multiple tags is formed by several identical data blocks in sequence, every data block follows the format stated below: ANT + LEN + EPC/TID + RSSI + Count; |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.3	ClearBuffer_G2

| Definition | int ClearBuffer_G2(BYTE *ComAdr, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Clear all the stored tag data from the memory buffer. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.4	GetBufferCnt_G2

| Definition | int GetBufferCnt_G2(BYTE *ComAdr, int *Count, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain the total tag amount stored in memory buffer. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | Count | int* | [out] | The total amount of tag stored in the memory buffer. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.5	 SetSaveLen

| Definition | int SetSaveLen(BYTE *ComAdr, BYTE SaveLen, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify the maximum EPC/TID length for inventory with memory buffer. This modification will clear all the tag data previously stored in the memory buffer. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | SaveLen | BYTE | [in] | 1 byte, the maximum EPC/TID length. 0x00 – 128bit length, i.e. 16 bytes. 0x01 – 496bit length, i.e. 62 bytes. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.3.5	GetSaveLen

| Definition | int GetSaveLen(BYTE *ComAdr, BYTE *SaveLen, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Load the configuration of maximum EPC/TID length for reader memory buffer |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | SaveLen | BYTE* | [out] | 1 byte, the maximum EPC/TID length. 0x00 – 128bit length, i.e. 16 bytes. 0x01 – 496bit length, i.e. 62 bytes. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

## 3.4	NXP Tag commands

3.4.1	SetPrivacyByEPC _G2

| Definition | int SetPrivacyByEPC_G2(BYTE * ComAddr, BYTE *EPC, BYTE ENum, BYTE *Password, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Setup read protection for a tag with particular EPC number; hence the tag will be unreadable by device. Device will not able to inquire EPC number of this tag via inventory operation. This command only valid for NXP UCODE EPC G2X tags. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | BYTE* | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | BYTE | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Password | BYTE* | [in] | 4 bytes, the access password. |
|  | MaskMem | BYTE | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | BYTE | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | BYTE* | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | int* | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.4.2	SetPrivacyWithoutEPC_G2

| Definition | int SetPrivacyWithoutEPC_G2(BYTE * ComAddr, BYTE *Password, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Setup read protection for tags within the effective field. Unlike the pervious command in 3.4.1, this command will perform operation on multiple inquired tags without tag identification. In order to perform operation on multiple tags, it is vital to keep access password consistent on those tags. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | Password | BYTE* | [in] | 4 bytes, the access password. |
|  | ErrorCode | int * | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.4.3	ResetPrivacy_G2

| Definition | int ResetPrivacy_G2(BYTE * ComAddr, BYTE *Password, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Unlock read protection of a tag. Only one tag is allowed to be place in the antenna effective area. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | Password | BYTE* | [in] | 4 bytes, the access password. |
|  | ErrorCode | int * | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.4.4	CheckPrivacy_G2

| Definition | int CheckPrivacy_G2(BYTE * ComAddr, BYTE * readpro, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is NOT ABLE TO identify if a specific tag supports the read protection function, the command ONLY inquire the read protection status of the tag. For tags with no read protection function, the default status will be unlocked. Only one tag is allowed to be place in the antenna effective area. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | readpro | BYTE* | [in] | 1 byte. 0x00 – read protection is disabled for the tag 0x01 – read protection is enabled for the tag Note For tags with no read protection function, the default status will be unlocked. |
|  | ErrorCode | int * | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.4.5	EASConfigure_G2

| Definition | int EASConfigure_G2(BYTE * ComAddr,BYTE *EPC,BYTE ENum,BYTE *Password,BYTE EAS, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, int *Errorcode,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify or reset EAS status. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | BYTE* | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | BYTE | [in] | 1 byte, the length of EPC number. For ENum = 255, mask will be enabled. |
|  | Password | BYTE* | [in] | 4 bytes, the access password.。 |
|  | EAS | BYTE | [in] | 1 byte. 0x00 – EAS alert is disabled; 0x01 – EAS alert is enabled. |
|  | MaskMem | BYTE | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | BYTE | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | BYTE* | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | int * | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.4.6	EASAlarm_G2

| Definition | int EASAlarm_G2(BYTE * ComAddr,int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Detect EAS alert. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | ErrorCode | int * | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

## 3.5	Impinj Monza4 QT tag commands

3.5.1	GetMonza4QTWorkParamter_G2

| Definition | int GetMonza4QTWorkParamter_G2(BYTE * ComAddr, BYTE *EPC, BYTE ENum, BYTE *Password, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, BYTE *QTcontrol, int *Errorcode, int FrmHandle) ; |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain the current working parameters of a tag. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | BYTE* | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | BYTE | [in] | 1 byte, the amount of words to be written. For ENum = 255, mask will be enabled. |
|  | Password | BYTE* | [in] | 4 bytes, the access password. |
|  | MaskMem | BYTE | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | BYTE | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | BYTE* | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | QTcontrol | BYTE* | [out] | Working parameters of tag. bit0: the current mirror page setting. 0 – private; 1 – public. bit1: distance protection setting. 0 – disabled; 1 – enabled. All other values are reserved. |
|  | ErrorCode | int * | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.5.2	SetMonza4QTWorkParamter_G2

| Definition | int SetMonza4QTWorkParamter_G2(BYTE * ComAddr, BYTE *EPC, BYTE ENum, BYTE QTcontrol, BYTE *Password, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify the current working parameters of a tag, only valid for Monza 4QT tags from Impinj. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | EPC | BYTE* | [in] | Tag EPC number, with a length stated in Enum. |
|  | ENum | BYTE | [in] | 1 byte, the amount of words to be written. For ENum = 255, mask will be enabled. |
|  | QTcontrol | BYTE | [in] | Working parameters of tag. bit0: the current mirror page setting. 0 – private; 1 – public. bit1: distance protection setting. 0 – disabled; 1 – enabled. All other values are reserved. |
|  | Password | BYTE* | [in] | 4 bytes, the access password. |
|  | MaskMem | BYTE | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | BYTE | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | BYTE* | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | ErrorCode | int * | [out] | If return value = 0xFC, ErrorCode is the error code returned from tag. Refer to Tag ErrorCode table for detail description. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.5.3	Inventory_QT_G2

| Definition | int Inventory_QT_G2(BYTE * ComAddr, BYTE QValue, BYTE Session, BYTE Target, BYTE InAnt,BYTE Scantime, BYTE FastFlag, BYTE *pEPCList, BYTE *Ant, int *Totallen, int *CardNum, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Inquire RFID tags with corresponding protocol within the effective field. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | QValue | BYTE | [in] | 1 byte. The original Q-value of EPC tag inventory. NOTE: 1. The setting of Q-value should follow the rule: 2Q ≈ total amount of tags within the effective field. The range of Q-value is 0 ~ 15, if other value is delivered in this field, reader will return a parameter error status in the response frame. |
|  | Session | BYTE | [in] | 1 byte, the Session-value of EPC tag inventory. 0x00 – apply S0 as Session value; 0x01 – apply S1 as Session value; 0x02 – apply S2 as Session value; 0x03 – apply S3 as Session value. Inventory for single tag or small amount of tag, S0 is the recommended setting. |
|  | Target | BYTE | [in] | 1 byte, the Target value of EPC tag inventory. 0x00 – apply Target A; 0x01 – apply Target B. |
|  | InAnt | BYTE | [in] | 1 byte, antenna selection for the current inventory. 0x80 – antenna 1; 0x81 – antenna 2; 0x82 – antenna 3; 0x83 – antenna 4; 0x84 – antenna 5; 0x85 – antenna 6; 0x86 – antenna 7; 0x87 – antenna 8; 0x88 – antenna 9; 0x89 – antenna 10; 0x8A – antenna 11; 0x8B – antenna 12. InAnt is 0x08 for single port reader. |
|  | Scantime | BYTE | [in] | 1 byte, the maximum operation time for inventory. The valid range of Scantime is 0 ~ 255, corresponding to (0 ~ 255)*100ms. For Scantime = 0, operation time is not limited. |
|  | FastFlag | BYTE | [in] | 1 byte, express inventory indicator 0 – disable express inventory, Target, InAnt and Scantime are NOT essential, can be set to default value 0; 1 – enable express inventory, Target, InAnt and Scantime are need to be defined. |
|  | pEPCList | BYTE* | [out] | The inquired tag data, the data block follows the format stated below: EPC/TID length + EPC/TID No. + RSSI. Data of multiple tags is formed by several identical data blocks in sequence |
|  | Ant | BYTE* | [out] | 1 byte indicates which antenna had inquired a tag. 4 antenna ports reader: Every bit in Ant represents one corresponding antenna. For example, 0x04 is 0000 0100 in binary. This indicates Antenna 3 had inquired this specific tag. 12 antenna ports reader Ant value 0 ~ 11 corresponding to Antenna 1 to Antenna 12. For example, Ant = 0 represents Antenna 1 had inquired this specific tag. |
|  | Totallen | int* | [out] | The total length of the received data stored in pEPCList. |
|  | CardNum | int* | [out] | The total amount of tag inquired during the current inventory. Note: Totallen and CardNum are 0 for call back return. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | 0x01: inventory completed, tag data will be delivered; 0x02: inventory timeout, operation is aborted, inquired tag data will be delivered. 0xF8: error occurred in antenna check, the target antenna might be disconnected. For description on other error, refer to reader error code table. |  |  |  |
| Sample code | N/A |  |  |  |

3.5.4	SetTagCustomFunction_G2

| Definition | int SetTagCustomFunction(BYTE * ComAddr, BYTE *InlayType, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | launch the customised utilities of some particular tags, to achieve specific tag function |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | InlayType | BYTE* | [in/out] | 1 byte, the type of tag. The valid value of this parameter is 0 ~ 254. 0 – tag type is not specified. 1 – launch the Peek function of Monza4QT tag (the status of tag will temporary change from public to private). Launching this function will make affection on the data reading/writing, block writing, protection word writing and EPC number writing operation. All other values are reserved. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

## 3.6	18000-6B Commands

3.6.1	InventorySingle_6B

| Definition | int InventorySingle_6B(BYTE *address, BYTE *Ant, BYTE* ID_6B, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Inquire a single tag. If multiple tags are placed within the effective field, reader may fail to inquire any tag. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | Ant | BYTE* | [out] | 1 byte. For example, 0x04 is 0000 0100 in binary, indicates Antenna 3 had inquired this specific tag. For 0x08, it is 0000 1000 in binary, indicates Antenna 4 had inquired tag information. |
|  | ID_6B | BYTE* | [out] | 10 bytes, the 1st byte is the length of UID, 0x08. The 2nd ~ 9th byte is the tag UID number, least-significant byte first. The 10th byte is RSSI. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.2	InventoryMultiple_6B

| Definition | int InventoryMultiple_6B(BYTE * ComAddr, BYTE Condition, BYTE StartAddress, BYTE mask, BYTE *ConditionContent, BYTE *Ant, BYTE* ID_6B, int *Cardnum, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Inquire tags according to specified condition. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | Condition | BYTE | [in] | the condition for inventory; 0x00 – equal to the reference; 0x01 – no equal to the reference; 0x02 – larger than the reference 0x03 – smaller than the reference |
|  | StartAddress | BYTE | [in] | 1 byte, entry address of the target. |
|  | mask | BYTE | [in] | 1 byte, specify the data for comparison. Every bit in mask is corresponding to a byte in ConditionContent. The most-significant bit (bit7) corresponds to the 1st byte in ConditionContent (from left to right). Consequently, the least- significant bit (bit0) corresponds to the last byte in ConditionContent (from left to right). ConditionContent is the reference data for comparison with data in tag starts from the location stated in StartAddress. The 1st byte of ConditionContent will be compare with the data store in Address, the last byte of ConditionContent will be compare with the data store in Address+7. Set the byte corresponding bit of Mask to 1, reader will perform comparison on that particular byte. Set the bit to 0, no comparison will be performed on that particular byte. |
|  | ConditionContent | BYTE* | [in] | 8 bytes, the value of reference. |
|  | Ant | BYTE* | [out] | 1 byte. For example, 0x04 is 0000 0100 in binary, indicates Antenna 3 had inquired this specific tag. For 0x08, it is 0000 1000 in binary, indicates Antenna 4 had inquired tag information. |
|  | ID_6B | BYTE* | [out] | Cardnum*10 bytes. UID of the tag. The length of every UID data block is 10 bytes. The 1st byte is the length value of UID number, i.e. 0x08. The 2nd ~ 9th bytes are the tag UID number. The 10th byte is the RSSI. |
|  | Cardnum | int* | [in] | 1 byte, the total amount of tag inquired during the current inventory. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | 0x15: inventory completed, tag data will be delivered; 0x16: inventory timeout, operation is aborted, inquired tag data will be delivered. 0xF8: error occurred in antenna check, the target antenna might be disconnected. For description on other error, refer to reader error code table. |  |  |  |
| Sample code | N/A |  |  |  |

3.6.3	ReadData_6B

| Definition | int ReadData_6B(BYTE * ComAddr, BYTE* ID_6B, BYTE StartAddress, BYTE Num, BYTE *Data, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Read multiple bytes from a specific address. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | ID_6B | BYTE* | [in] | 8 bytes, UID of the target tag. |
|  | StartAddress | BYTE | [in] | 1 byte, the entry address of target. The valid value range of StartAddress is 0 ~ 223. Reader will return a parameter error status in the response frame if incorrect address is delivered. |
|  | Num | BYTE | [in] | 1 byte, the amount of words to be read in this operation. The valid value range of Num is 1 ~ 32. If the sum of StartAddress and Num is larger than 224 or Num is equal to 0 or larger than 32, reader will return a parameter error status in the response frame. |
|  | Data | BYTE* | [out] | Inquired tag data, with a length stated in Num. |
|  | Errorcode | int* | [out] | 1 byte, reserved. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.4	WriteData_6B

| Definition | int WriteData_6B(BYTE * ComAddr, BYTE *ID_6B, BYTE StartAddress, BYTE *Writedata,BYTE Writedatalen, BYTE *writtenbyte, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Write multiple bytes to a specific tag. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | ID_6B | BYTE* | [in] | 8 bytes, UID of the target tag. |
|  | StartAddress | BYTE | [in] | 1 byte, the entry address of target. The valid value range of StartAddress is 0 ~ 223. Reader will return a parameter error status in the response frame if incorrect address is delivered. |
|  | Writedata | BYTE* | [in] | Data to be written to tag, with a length stated in Writedatalen. |
|  | Writedatalen | BYTE | [in] | 1 byte, the amount of words to be written in this operation. |
|  | writtenbyte | BYTE* | [out] | 1 byte, reserved. |
|  | Errorcode | int* | [out] | 1 byte, reserved. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.5	CheckLock_6B

| Definition | int CheckLock_6B(BYTE * ComAddr, BYTE *ID_6B, BYTE StartAddress, BYTE *ReLockState, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain the lock status of a specific byte. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | ID_6B | BYTE* | [in] | 8 bytes, UID of the target tag. |
|  | StartAddress | BYTE | [in] | 1 byte, the entry address of target. The valid value range of StartAddress is 0 ~ 223. Reader will return a parameter error status in the response frame if incorrect address is delivered. |
|  | ReLockState | BYTE* | [out] | 1 byte. 0x00: the byte is unlocked; 0x01: the byte is already locked. |
|  | Errorcode | int* | [out] | 1 byte, reserved. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.6.6	Lock_6B

| Definition | int Lock_6B(BYTE * ComAddr, BYTE *ID_6B, BYTE StartAddress, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Lock a specific byte in a tag. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | ID_6B | BYTE* | [in] | 8 bytes, UID of the target tag. |
|  | StartAddress | BYTE | [in] | 1 byte, the entry address of target. The valid value range of StartAddress is 0 ~ 223. Reader will return a parameter error status in the response frame if incorrect address is delivered. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

## 3.7	Reader customised commands

3.7.1	GetReaderInformation

| Definition | int GetReaderInformation(BYTE* ComAdr,BYTE* VersionInfo, BYTE* ReaderType, BYTE* TrType, BYTE* dmaxfre, BYTE* dminfre, BYTE* powerdBm, BYTE* ScanTime, BYTE*Ant, BYTE*BeepEn, BYTE*OutputRep, BYTE*CheckAnt, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain reader information, including reader address (ComAdr), firmware version (VersionInfo) and other reader information. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands.。 |
|  | VersionInfo | BYTE* | [out] | 2 bytes, firmware version. The 1st byte: the main version number; The 2nd byte: the subversion number. |
|  | ReaderType | BYTE* | [out] | 1 byte. Reader Type code. |
|  | TrType | BYTE* | [out] | 1 byte, reader supported protocol. |
|  | dmaxfre | BYTE* | [out] | 1 byte. bit7 ~ bit6: frequency band configuration; bit5 ~ bit0: current maximum frequency point. |
|  | dminfre | BYTE* | [out] | 1 byte. bit7 ~ bit6: frequency band configuration; bit5 ~ bit0: current minimum frequency point. |
|  | powerdBm | BYTE* | [out] | 1 byte, Output RF power. The range is 0 ~ 33 and the unit is dBm. |
|  | ScanTime | BYTE* | [out] | 1 byte, inventory time. Reader will respond to the inventory command delivered from host within this specific inventory time. |
|  | Ant | BYTE* | [out] | 1 byte, antenna configuration. |
|  | BeepEn | BYTE* | [out] | 1 byte, buzzer status information. |
|  | OutputRep | BYTE* | [out] | 1 byte, GPIO status for notification. |
|  | CheckAnt | BYTE* | [out] | 1 byte, Antenna check configuration. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

Frequency band configuration table

| MaxFre (bit7) | MaxFre (bit6) | MinFre (bit7) | MinFre (bit6) | FreqBand |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | Chinese band2 |
| 0 | 0 | 1 | 0 | US band |
| 0 | 0 | 1 | 1 | Korean band |
| 0 | 1 | 0 | 0 | EU band |
| 0 | 1 | 1 | 0 | Ukraine band |
| 1 | 0 | 0 | 0 | Chinese band1 |
| 1 | 1 | 0 | 0 | US band3 |

3.7.2	SetAddress

| Definition | int SetAddress(BYTE * ComAddr, BYTE ComAdrData, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Reader will apply the user provided value as its reader address and write this value to EEPROM for preservation purpose. Default setting of reader address is 0x00, the valid value of the setting is 0x00 ~ 0x0FE. Reader address will be reset to 0x00, if the provided address value is 0xFF. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | ComAdrData | BYTE | [in] | 1 byte, the new address setting. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.3	SetInventoryScanTime

| Definition | int SetInventoryScanTime(BYTE *address, BYTE ScanTime, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify maximum operation time for inventory command. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | ScanTime | BYTE | [in] | 1 byte, operation time for inventory. The acceptable range of Scantime is 0 ~ 255, corresponding to (0 ~ 255)*100ms. For Scantime = 0, operation time is not limited until a whole reading cycle is completed. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.4	SetRfPower

| Definition | int SetRfPower(BYTE *address, BYTE PowerDbm, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify reader RF power |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | PowerDbm | BYTE | [in] | 1 byte, reader RF power. The range is 0 ~ 33 and the unit is dBm. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.5	SetRegion

| Definition | int SetRegion(BYTE *address, BYTE dmaxfre, BYTE dminfre, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Select maximum and minimum frequency. Maximum frequency must be larger or equal the minimum frequency. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | dmaxfre | BYTE | [in] | 1 byte. bit7 ~ bit6: frequency band configuration; bit5 ~ bit0: maximum frequency point. |
|  | dminfre | BYTE | [in] | 1 byte. bit7 ~ bit6: frequency band configuration; bit5 ~ bit0: minimum frequency point. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table) |  |  |  |
| Sample code | N/A |  |  |  |

Frequency band configuration table

| MaxFre (bit7) | MaxFre (bit6) | MinFre (bit7) | MinFre (bit6) | FreqBand |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | Chinese band2 |
| 0 | 0 | 1 | 0 | US band |
| 0 | 0 | 1 | 1 | Korean band |
| 0 | 1 | 0 | 0 | EU band |
| 0 | 1 | 1 | 0 | Ukraine band |
| 1 | 0 | 0 | 0 | Chinese band1 |
| 1 | 1 | 0 | 0 | US band3 |

Formulae for different frequency bands:

Chinese band2: Fs = 920.125 + N * 0.25 (MHz) where N∈ [0, 19]. US band: Fs = 902.75 + N * 0.5 (MHz) where N∈ [0, 49]. Korean band: Fs = 917.1 + N * 0.2 (MHz) where N∈ [0, 31]. EU band: Fs = 865.1 + N*0.2(MHz) where N∈ [0, 14]. Ukraine band: Fs = 868.0 + N*0.1(MHz) where N∈ [0, 6]. Chinese band1: Fs = 840.125 + N * 0.25 (MHz) where N∈ [0, 19]. US band3: Fs = 902 + N * 0.5 (MHz) where N∈ [0, 52].

3.7.6	SetBaudRate

| Definition | int SetBaudRate(BYTE * ComAddr, BYTE baud, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify serial baud rate. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | baud | BYTE | [in] | 1 byte, serial baud rate. 0 – 9600bps; 1 – 19200bps; 2 – 38400bps; 5 – 57600bps; 6 – 115200bps. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.7	BuzzerAndLEDControl

| Definition | int BuzzerAndLEDControl(BYTE * ComAddr, BYTE AvtiveTime, BYTE SilentTime, BYTE Times, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Control the TAG LED and buzzer to perform specific action. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | AvtiveTime | BYTE | [in] | 1 byte, TAG LED and buzzer switch ON time (ActiveT*50ms). The range of ActiveT is 0 ~ 255. Default setting is 0. |
|  | SilentTime | BYTE | [in] | 1 byte, TAG LED and buzzer switch OFF time (SilentT*50ms). The range of SilentT is 0 ~ 255. Default setting is 0. |
|  | Times | BYTE | [in] | TAG LED and buzzer action time, The range of Times is 0 ~ 255. Default setting is 0. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.8	SetAntennaMultiplexing

| Definition | int SetAntennaMultiplexing(BYTE *ComAdr, BYTE Ant, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify antenna configuration of 4 ports reader. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | Ant | BYTE | [in] | 1 byte, antenna configuration information. 1 antenna is the MINIMUM requirement for operation. bit0: antenna 1 configuration bit. 0 – disable antenna 1; 1 – enable antenna 1. bit1: antenna 2 configuration bit. 0 – disable antenna 2; 1 – enable antenna 2. bit2: antenna 3 configuration bit. 0 – disable antenna 3; 1 – enable antenna 3. bit3: antenna 4 configuration bit. 0 – disable antenna 4; 1 – enable antenna 4. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.9	SetAntenna

| Definition | int SetAntenna(BYTE *ComAdr, BYTE SetOnce, BYTE AntCfg1, BYTE AntCfg2, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify antenna configuration of 8 or 16 ports reader. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | SetOnce | BYTE | [in] | 1 byte, configuration preservation status during power off. 0x00 – configuration preserved during reader power off; 0x01 – configuration is not preserved. |
|  | AntCfg1 | BYTE | [in] | 1 byte, antenna configuration information. bit0: antenna 9 configuration bit. 0 – disable antenna 9; 1 – enable antenna 9. bit1: antenna 10 configuration bit. 0 – disable antenna 10; 1 – enable antenna 10. bit2: antenna 11 configuration bit. 0 – disable antenna 11; 1 – enable antenna 11. bit3: antenna 12 configuration bit. 0 – disable antenna 12; 1 – enable antenna 12. bit4: antenna 13 configuration bit. 0 – disable antenna 13; 1 – enable antenna 13. bit5: antenna 14 configuration bit. 0 – disable antenna 14; 1 – enable antenna 14. bit6: antenna 15 configuration bit. 0 – disable antenna 15; 1 – enable antenna 15. bit7: antenna 16 configuration bit. 0 – disable antenna 16; 1 – enable antenna 16. |
|  | AntCfg2 | BYTE | [in] | 1 byte, antenna configuration information. bit0: antenna 1 configuration bit. 0 – disable antenna 1; 1 – enable antenna 1. bit1: antenna 2 configuration bit. 0 – disable antenna 2; 1 – enable antenna 2. bit2: antenna 3 configuration bit. 0 – disable antenna 3; 1 – enable antenna 3. bit3: antenna 4 configuration bit. 0 – disable antenna 4; 1 – enable antenna 4. bit4: antenna 5 configuration bit. 0 – disable antenna 5; 1 – enable antenna 5. bit5: antenna 6 configuration bit. 0 – disable antenna 6; 1 – enable antenna 6. bit6: antenna 7 configuration bit. 0 – disable antenna 7; 1 – enable antenna 7. bit7: antenna 8 configuration bit. 0 – disable antenna 8; 1 – enable antenna 8. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.10	SetBeepNotification

| Definition | int SetBeepNotification(BYTE *ComAdr,BYTE BeepEn, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Enable/disable buzzer. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | BeepEn | BYTE | [in] | 1 byte. 0x00 – disable buzzer; 0x01 – enable buzzer. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.11	SetReal_timeClock

| Definition | int SetReal_timeClock(BYTE *ComAdr, BYTE *paramer, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Adjust real time clock. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | paramer | BYTE* | [in] | 6 bytes, including Year, Month, Day, Hour, Minute and Second. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.12	GetTime

| Definition | int GetTime(BYTE *ComAdr, BYTE *paramer, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Get current time from real time clock. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | paramer | BYTE* | [out | 6 bytes, including Year, Month, Day, Hour, Minute and Second. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.13	SetRelay

| Definition | int SetRelay(BYTE *ComAdr, BYTE RelayTime, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Control the state of the built-in relay. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | RelayTime | BYTE | [in] | Configuration of the relay pickup time, the valid range of the parameter is 1 ~ 255, corresponds to (1 ~ 255)*50ms |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.14	GetGPIOStatus

| Definition | int GetGPIOStatus(BYTE *ComAdr, BYTE *OutputPin, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain the input and output states of GPIO. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | OutputPin | BYTE* | [out] | 1 byte, bit0 ~ bit3 represent IN1 to IN4 and bit4 ~ bit7 represent Out1 to Out4 respectively. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.15	SetGPIO

| Definition | int SetGPIO(BYTE *ComAdr, BYTE OutputPin, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Control the output of GPIO. The default output is high TTL level. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | OutputPin | BYTE | [in] | 1 byte, the output state of GPIO (pin Out1 to Out4). Bit0 ~ bit3 control Out1 to Out4 respectively. Bit4 ~ bit7 are reserved. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.16	SetNotificationPulseOutput

| Definition | int SetNotificationPulseOutput(BYTE *ComAdr, BYTE OutputRep, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Enable / disable a 2ms low level notification pulse on the associated port for every tag detection. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | OutputRep | BYTE | [in] | 1 byte. bit0: associated with antenna 1 and Out1 pin. 1 – Out1 will output a 2ms low level notification pulse for every tag detection from antenna 1; 0 – disable notification signal. bit1: associated with antenna 2 and Out2 pin. 1 – Out2 will output a 2ms low level notification pulse for every tag detection from antenna 2; 0 – disable notification signal. bit2: associated with antenna 3 and Out3 pin. 1 – Out3 will output a 2ms low level notification pulse for every tag detection from antenna 3; 0 – disable notification signal. bit3: associated with antenna 4 and Out4 pin. 1 – Out4 will output a 2ms low level notification pulse for every tag detection from antenna 4; 0 – disable notification signal. All other values are reserved, default is 0. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.17	SetTIDParameter

| Definition | int SetTIDParameter(BYTE * ComAddr, BYTE AdrTID, BYTE LenTID, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify the parameter for TID inventory |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | AdrTID | BYTE | [in] | 1 byte, entry address of TID memory inventory. |
|  | LenTID | BYTE | [in] | 1 byte, data length for TID inventory operation, the valid range of LenTID is 0 ~ 15. Reader will return a parameter error status in the response frame if incorrect value is delivered in this field. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.18	SetQS

| Definition | int SetQS(BYTE *address, BYTE Qvalue, BYTE Session, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify parameters for multiple-tag inventory in auto-mode. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | Qvalue | BYTE | [in] | 1 byte, the Q-value of inventory. The valid range of Q-value is 0 ~ 15. |
|  | Session | BYTE | [in] | 1 byte, the Session-value of inventory. 0x00 – apply S0 as Session value; 0x01 – apply S1 as Session value; 0x02 – apply S2 as Session value; 0x03 – apply S3 as Session value. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.19	GetQS

| Definition | int GetQS(BYTE * ComAddr, BYTE* Qvalue, BYTE* Session, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain configuration for multiple-tag inventory in auto-mode. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | Qvalue | BYTE* | [out] | 1 byte, the Q-value of inventory. |
|  | Session | BYTE* | [out] | 1 byte, the Session-value of inventory. 0x00 – applied S0 as Session value; 0x01 – applied S1 as Session value; 0x02 – applied S2 as Session value; 0x03 – applied S3 as Session value. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.20	GetSeriaNo

| Definition | int GetSeriaNo(BYTE * ComAddr, BYTE* SeriaNo, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain 4 bytes reader unique serial number. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | SeriaNo | BYTE* | [out] | 4 bytes, the unique serial number of reader. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.21	SetCheckAnt

| Definition | int SetCheckAnt(BYTE *address, BYTE CheckAnt, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Enable antenna check during tag reading/writing operations. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | CheckAnt | BYTE | [in] | 1 byte, antenna check switch. 0x00 – disable antenna check; 0x01 – enable antenna check. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.22	SetWorkMode

| Definition | int SetWorkMode(BYTE * ComAddr, BYTE Read_mode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Set reader working mode (only supported by UHFReader288 and UHFReader288ADV). |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | Read_mode | BYTE | [in] | 1 byte, working mode. 0x00 – answer mode; 0x01 – auto mode; 0x02 – trigger mode(low level active); 0x03 – trigger mode(high level active); 0x06 – real time mode. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.23	GetSystemParameter

| Definition | int GetSystemParameter(BYTE * ComAddr, BYTE *Read_mode, BYTE *Accuracy, BYTE *RepCondition, BYTE *RepPauseTime, BYTE *ReadPauseTim, BYTE *TagProtocol, BYTE *MaskMem, BYTE *MaskAdr, BYTE *MaskLen, BYTE *MaskData, BYTE *TriggerTime, BYTE *AdrTID, BYTE *LenTID, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain auto mode parameters (only supported by UHFReader288, UHFReader288ADV). |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | Read_mode | BYTE* | [out] | 1 byte, working mode information. |
|  | Accuracy | BYTE* | [out] | 1 byte, accuracy parameter of EAS check in auto-mode. |
|  | RepCondition | BYTE* | [out] | 1 byte, parameter of answering form in auto-mode. |
|  | RepPauseTime | BYTE* | [out] | 1 byte, parameter of answering form in auto-mode. |
|  | ReadPauseTim | BYTE * | [out] | 1 byte, time break between two inventories in auto-mode. |
|  | TagProtocol | BYTE * | [out] | 1 byte, tag type for auto-mode inventory. |
|  | MaskMem | BYTE* | [out] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | BYTE* | [out] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | BYTE* | [out] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | BYTE* | [out] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | TriggerTime | BYTE* | [out] | 1 byte, the effective trigger time in trigger mode. |
|  | AdrTID | BYTE* | [out] | 1 byte, initial byte address for TID memory read operation. |
|  | LenTID | BYTE* | [out] | 1 byte, data length for the TID read operation. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.24	ReadActiveModeData

| Definition | int ReadActiveModeData(BYTE * LoadData, int *Datalength, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain reader uploaded data in auto-mode or real time mode. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | LoadData | BYTE* | [out] | Reader uploaded raw data, with a length stated in Datalength. Refer to user manual for detail format. |
|  | Datalength | BYTE* | [out] | Byte length of LoadData. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.25	SetEASSensitivity

| Definition | int SetEASSensitivity(BYTE *ComAdr, BYTE Accuracy, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Read data from a specified data block, and obtain the block EAS parameter byte. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | Accuracy | BYTE | [in] | 1 byte. bit7: 1 – close relay for 100ms once EAS is detected in auto-mode; 0 – relay remains in current state. bit6 ~ bit0: the valid range is 0 ~ 8, lower value represent higher sensitivity but lower accuracy. Default setting is 8, i.e. highest accuracy and lowest sensitivity. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.26	SetMask

| Definition | int SetMask(BYTE *ComAdr, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify the parameters of G2 tag inventory in auto-mode. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | MaskMem | BYTE | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | BYTE | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | BYTE* | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0.。 |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.27	SetResponsePamametersofAuto_runningMode

| Definition | int SetResponsePamametersofAuto_runningMode(BYTE *ComAdr, BYTE RepCondition, BYTE RepPauseTime,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify reader answering form in auto-mode. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE * | [in/out] | Reader address, obtained from open port commands. |
|  | RepCondition | BYTE | [in] | 1 byte, options for answering form in auto-mode. 0x00 – notify upon command. Reader will only produce tag information report upon notification command reception; 0x01 – notify over time. Reader will produce tag information report in every RepPauseTime time interval; 0x02 – notify upon new tag. Reader will produce tag information report and store the information once a new tag is inquired; 0x03 – notify upon tag information removal. Reader will produce tag information report upon tag information deletion due to lack of memory. 0x04 – notify upon tag information changes. Reader will produce tag information report upon tag information adding or removal in the memory. All other values are reserved, reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | RepPauseTime | BYTE | [in] | 1 byte, the time break between two tag information reports, in notify over time form. The range of RepPauseTime is (0 ~ 255)*1s. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.28	SetInventoryInterval

| Definition | int SetInventoryInterval(BYTE *ComAdr, BYTE ReadPauseTim, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify the inventory time break in auto-mode. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE * | [in/out] | Reader address, obtained from open port commands. |
|  | ReadPauseTim | BYTE | [in] | 1 byte, time break between 2 real time inventories. 0x00 – 10ms; 0x01 – 20ms; 0x02 – 30ms; 0x03 – 50ms; 0x04 – 100ms. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.29	SelectTagType

| Definition | int SelectTagType(BYTE *ComAdr, BYTE TagProtocol, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify the inventory tag type in auto-mode. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE * | [in/out] | Reader address, obtained from open port commands. |
|  | TagProtocol | BYTE | [in] | 1 byte. bit7: tag protocol type. 0 – set inventory on G2(18000-6C) tag; for TagProtocol = 0x00, only inquired EPC of G2 tags; for TagProtocol = 0x02, only detects EAS; for TagProtocal = 0x10, only inquired TID of G2 tags. 1 – for TagProtocol = 0x80, set inventory on 18000-6B tag. All other value reserved. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.30	GetTagBufferInfo

| Definition | int GetTagBufferInfo(BYTE *ComAdr, BYTE *Data, int *dataLength, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain all the tag information from reader memory. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | Data | BYTE* | [out] | Reader stored data, with a length stated in Datalength. |
|  | dataLength | int* | [out] | Byte length of Data. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.31	ClearTagBuffer

| Definition | int ClearTagBuffer(BYTE *ComAdr,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Clear all the stored tag data from the memory buffer |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.32	GetModuleVersion

| Definition | int GetModuleVersion(BYTE * ComAddr, BYTE *Version, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain reader internal R2000 module firmware version. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | Version | BYTE* | [out] | 2 bytes, R2000 module version number. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.33	SetFlashRom

| Definition | int SetFlashRom(BYTE * ComAddr, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Set the reader to R2000 update mode. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.34	WriteRfPower

| Definition | int WriteRfPower(BYTE * ComAddr, BYTE PowerDbm, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify RF power configuration separately for write operations |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | PowerDbm | BYTE | [in] | 1 byte, separate reader RF power for write operation. The range is 0 ~ 33 and the unit is dBm. bit7 = 0: disable; bit7 = 1: enable. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.35	ReadRfPower

| Definition | int WriteRfPower(BYTE * ComAddr, BYTE PowerDbm, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain RF power configuration for write operations |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | PowerDbm | BYTE* | [out] | 1 byte, separate reader RF power for write operation. The range is 0 ~ 33 and the unit is dBm. bit7 = 0: disabled; bit7 = 1: enabled. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.36	RetryTimes

| Definition | int RetryTimes(BYTE * ComAddr, BYTE *Times, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify or load the maximum operation retry time configuration |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | Times | BYTE* | [out] | 1 byte. bit7 = 1: modify maximum write retry time; bit7 = 0: load maximum write retry time The valid range of configuration is 0 ~ 7. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.37	SetReadMode

| Definition | int SetReadMode(BYTE *ComAdr,BYTE ReadMode ,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Select reader working mode. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | ReadMode | BYTE | [in] | 1 byte. 0x00 – answer mode; 0x01 – real time inventory mode; 0x02 – trigger mode. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.38	SetReadParameter

| Definition | int SetReadParameter(BYTE *ComAdr, BYTE *Parameter, BYTE MaskMem, BYTE *MaskAdr, BYTE MaskLen, BYTE *MaskData, BYTE MaskFlag, BYTE AdrTID, BYTE LenTID, BYTE TIDFlag, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Modify the real time inventory associated parameters. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | Parameter | BYTE* | [in] | Parameters definition 5 bytes. Parameter[0]: 1 byte, tag type definition for real time inventory. 0 – EPC C1G2 (ISO18000-6C) tags. 1 – ISO18000-6B tags. All other values are reserved, reader will return a parameter error status in the response frame if other value is delivered in this field. Parameter[1]: 1 byte, time break between 2 real time inventories. 0x00 – 10ms; 0x01 – 20ms; 0x02 – 30ms; 0x03 – 50ms; 0x04 – 100ms. Parameter[2]: 1 byte, tag filtering time. The valid value of this parameter is 0 ~ 255, corresponds to (0 ~ 255)*1s. In real time inventory, if reader detects a particular tag for more than 1 time, reader will only upload tag information of this tag once within the pre-defined filtering time. For FliterTime = 0, disable tag filtering function. Parameter[3]: 1 byte, the original Q-value of the EPC tag inventory, the setting of Q-value should follows the rule: 2Q ≈ total amount of tags within the effective field. The range of Q-value is 0 ~ 15, if other value is delivered in this field, reader will return a parameter error status in the response frame. Parameter[4]: 1 byte, the Session-value of the EPC tag inventory. 0x00 – apply S0 as Session value; 0x01 – apply S1 as Session value; 0x02 – apply S2 as Session value; 0x03 – apply S3 as Session value; 0xff – apply reader smart configuration (only valid in EPC inventory). |
|  | MaskMem | BYTE | [in] | 1 byte, mask area indication. 0x01 – EPC memory; 0x02 – TID memory; 0x03 – User memory. |
|  | MaskAdr | BYTE* | [in] | 2 bytes, entry bit address of the mask. The valid range of MaskAdr is 0 ~ 16383. |
|  | MaskLen | BYTE | [in] | 1 byte, bit length of mask (unit: bits). |
|  | MaskData | BYTE* | [in] | N bytes, mask data. N = MaskLen/8. If MaskLen is not a multiple of 8 integer, N= int[MaskLen/8]+1. Non-specified lower significant figures should be filled up with 0. |
|  | AdrTID | BYTE | [in] | 1 byte, entry address of TID memory inventory. |
|  | LenTID | BYTE | [in] | 1 byte, data length for TID inventory operation, the valid range of LenTID is 0 ~ 15. |
|  | TIDFlag | BYTE | [in] | 1 byte, inventory purpose indicator. 0 – EPC inventory; 1 – TID inventory. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.39	GetReadParameter

| Definition | int GetReadParameter (BYTE *ComAdr,BYTE* Parameter,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Obtain the real time inventory associated parameters. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | Parameter | BYTE* | [out] | Parameters definition ReadMode: 1 byte, the current working mode of reader. TagProtocol: 1 byte, tag type definition of real time inventory. ReadPauseTime: 1 byte, time break between 2 real time inventories. FliterTime: 1 byte, tag filtering time of real time inventory. QValue: 1 byte, the original Q-value of real time mode for EPC inventory. Session: 1 byte, the Session-value of real time mode for EPC inventory. MaskMem, MaskAdr, MaskLen, MaskData: mask condition for EPC C1G2 tag inventory in real time mode. MaskMem and MaskLen are 1 byte long respectively. MaskAdr is 2 bytes long, most-significant byte first. MaskData has constant length of 32 bytes, fill the over MaskLen value content with 0. AdrTID: entry address of TID memory inventory. LenTID: data length for TID inventory operation. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.40	SetRfPowerByAnt

| Definition | int SetRfPower(BYTE *address, BYTE* PowerDbm, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Set the individual power of each antenna of the reader. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | PowerDbm* | BYTE* | [in] | 4 bytes, reader power, byte0-byte3 represent the power of antenna 1-antenna 4 respectively. Bit0~Bit6 of each byte, the antenna power parameter to be set, the range is 0~30, when the value is 30, the output power is about 1W. Bit7: Indicates whether the current configuration is saved after power-off. Bit7=0 save when power off; Bit7=1 not save when power off. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.41	GetRfPowerByAnt

| Definition | int GetRfPower(BYTE *address, BYTE* PowerDbm, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Get the individual power of each antenna of the reader. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE* | [in/out] | Reader address, obtained from open port commands. |
|  | PowerDbm* | BYTE* | [out] | 4 bytes, reader power, byte0-byte3 represent the power of antenna 1-antenna 4 respectively. Bit0~Bit6 of each byte, the antenna power parameter to be set, the range is 0~30, when the value is 30, the output power is about 1W. Bit7: Indicates whether the current configuration is saved after power-off. Bit7=0 save when power off; Bit7=1 not save when power off. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.42	SetCfgParameter

| Definition | int SetCfgParameter(BYTE * ComAddr, BYTE opt, BYTE cfgNum, BYTE * data,int len, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to configure extended parameters, See user manual for details. |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE * | [in/out] | Reader address, obtained from open port commands. |
|  | opt | BYTE | [in] | 0-save;1-not save; |
|  | cfgNum | BYTE | [in] | Used to specify the parameter sequence number to be set in the command data block |
|  | data | BYTE * | [in] | Set the parameter content, see the reader parameter configuration table for details. |
|  | len | int | [in] | Parameter content length. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.43	GetCfgParameter

| Definition | int GetCfgParameter (BYTE *ComAddr, BYTE cfgNum,BYTE * data, int* len, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to read extended parameters. See user manual for details |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE * | [in/out] | Reader address, obtained from open port commands. |
|  | cfgNum | BYTE | [in] | Used to specify the parameter sequence number to be get in the command data block |
|  | data | BYTE * | [out] | Read the parameter content, see the reader parameter configuration table for details |
|  | len | ref int | [out] | Parameter content length. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.44	SetProfile

| Definition | int SetProfile (BYTE *ComAddr, BYTE* Profile, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to configure the profile, |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE * | [in/out] | Reader address, obtained from open port commands. |
|  | Profile | BYTE * | [in/out] | profile number configuration. bit7: action on profile number configuration. 0 – load profile number; 1 – modify profile number. Bit6:Power Down Save Flag (Ex10 effective) 0 – save； 1 – not save。 Bit5 ~ bit0: profile number configuration. For bit7 = 0, bit6 ~ bit0 will be ignored. For bit7 = 1, bit5 ~ bit0 is the new configuration of profile number. All other values are reserved, reader will return a parameter error status in the response frame if other value is delivered in this field. R2000： Profle 0：Tari 25uS，FM0 40KHz； Profile1：Tari 25uS，Miller 4 250KHz Profile2：Tari 25uS，Miller 4 300KHz； Profile3：Tari 6.25uS，FM0 400KHz。 Ex10： Profile11: 640kHZ, FM0 ,Tari 7.5 us Profile1: 640kHZ, Miller2,Tari 7.5 us Profile15: 640kHZ, Miller4,Tari 7.5 us Profile12: 320kHZ, Miller2,Tari 15 us Profile3: 320kHZ, Miller2,Tari 20 us Profile5: 320kHZ, Miller4,Tari 20 us Profile7: 250kHZ, Miller4,Tari 20 us Profile13: 160kHZ, Miller8,Tari 20 us Profile50: 640kHZ, FM0 ,Tari 6.25 us Profile51: 640kHZ, Miller2,Tari 6.25 us Profile52: 426kHZ, FM0 ,Tari 15 us Profile53: 640kHZ, Miller4,Tari 7.5 us |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

3.7.45	StopImmediately

| Definition | int StopImmediately (BYTE *ComAddr, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| Description | This command is used to immediately stop the command currently being executed by the reader and return the current command reply. This command only applies to the command to immediately stop querying labels (command code 0x01) |  |  |  |
| Parameters | Name | Type | Orientation | Notes |
|  | ComAddr | BYTE * | [in/out] | Reader address, obtained from open port commands. |
|  | FrmHandle | int | [in] | Reader connected handle, obtained from open port commands. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |  |
| Sample code | N/A |  |  |  |

# Appendix 1: Reader Error code table

| Error code | Description |
| --- | --- |
| 0x00 | API is called successfully. |
| 0x01 | Tag inventory is completed successfully and reader is able to deliver data response within the predefined inventory time. |
| 0x02 | Inventory timeout. |
| 0x05 | Access password error. |
| 0x09 | Kill password error. |
| 0x0A | All-zero tag killing password is invalid. |
| 0x0B | Command is not support by the tag |
| 0x0C | All-zero tag access password is invalid for such command. |
| 0x0D | Fail to setup read protection for a protection enabled tag. |
| 0x0E | Fail to unlock a protection disabled tag. |
| 0x10 | Some bytes stored in the tag are locked. |
| 0x11 | Lock operation failed. |
| 0x12 | Already locked, lock operation failed. |
| 0x13 | Fail to store the value of some preserved parameters. Configuration will still valid before reader shut down. |
| 0x14 | Modification failed. |
| 0x15 | Response within the predefined inventory time. |
| 0x17 | Further data is waiting to be delivered. |
| 0x18 | Reader memory is full. |
| 0x19 | All-zero access password is invalid for such command or command is not supported by the tag. |
| 0xF8 | Error detected in antenna check. |
| 0xF9 | Operation failed. |
| 0xFA | Tag is detected, but fails to complete operation due to poor communication. |
| 0xFB | No tag is detected. |
| 0xFC | Error code returned from tags. |
| 0xFD | Command length error. |
| 0xFE | Illegal command. |
| 0xFF | Parameter error. |
| 0x30 | Communication error. |
| 0x33 | Reader is busy, operation in process. |
| 0x35 | Port is already opened. |
| 0x37 | Invalid handle. |

# Appendix 2: Tag ErrorCode table

| ErrorCode | Description |
| --- | --- |
| 0x00 | Other errors, all other Errorcode non-specified error. |
| 0x03 | Memory overload, location is not found or unsupported PC value. |
| 0x04 | Memory is being temporarily / permanently locked, unable to perform write operation. |
| 0x0B | Unable to perform write operation due to insufficient power supply to tag. |
| 0x0F | Undefined or tag unsupported errors. |